- OBJECTIVE:

To lay down procedure of sampling of raw material in order to get a representative sample of the whole lot for analysis.

- SCOPE:

This procedure is applicable for sampling of all raw materials (solids and liquids) and raw materials due for retesting at Biomedical Ltd.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Quality Control Analyst-responsible for the preparation of new or revision of existing SOPs

- ﻿﻿ QC Head- responsible of reviewing the SOPs

- QA Manager – responsible for the approval of the SOP

- Doc. Unit Head – responsible for the distribution and control of SOPs to various departments

- ABBREVIATION AND DEFINITIONS

COA = Certificate of Analysis.

O.C. = Quality Control

No. = Number 6.4 Mfg. date = Manufacturing date

Exp. Date = Expiry date

MRR = Material Receiving Report

UT = Under Test

AR.No. = Analytical Report Number

API = Active Pharmaceutical Ingredients

CC= Change control

PPE’S = Personnel protective equipment

RMS= Raw material store

HEPA= High efficiency particulate Air

STP =Standard testing procedure

DEFINITION:

Sampling: is the regulated, systematic collection of a representative portion of raw materials, packaging components, or finished products to evaluate their quality, safety, and compliance with specifications

- PROCEDURES

Precaution

- ﻿﻿Use sampling device after mopping with freshly prepared 70% IPA for collecting the sample for microbiological limit test.

- ﻿﻿Material of only one consignment shall be sampled at a time.

- ﻿﻿Use different sampling devices/ material for sampling of different RM & PM.

- ﻿During sampling use PPE such as mask, cap, hand gloves and factory shoe.

- ﻿﻿Switch on the RLAF 15minutes before sampling

- ﻿﻿Carry out, sampling of material in sampling booth when is on

- ﻿﻿Ensure that the manometer reading of sampling laminar airflow unit is between 7 to 15 mm of water before sampling.

- Perform the hygroscopic Raw Material in humidity controlled area.

- Verify the humidity of the sampling room through a hygrometer in the sampling area.

- In case the humidity level goes out of the limit, inform the maintenance to rectify it

- Perform the sampling of hygroscopic raw materials immediately after the opening of the container and close the container immediately after sampling.

- ﻿All solids as well as liquids materials except solvents are to be sampled in sampling booth.

- ﻿﻿Sample for microbial testing shall be taken first followed by other sampling.

- ﻿﻿Make sure that cleaning status on room label is available on the sampling booth before starting sampling.

- Any spillage during the sampling should be cleaned immediately.

General procedure

- Receive the MATERIAL RECEIVING REPORT with sign of store in-charge and enter the following details in the raw material log register maintained separately in Q.C. dept. raw material, company wise, Date of Receipt, Supplied By, A.R. No., Qty received, Batch No., Mfg Date, Name of Material, Exp. Date, Manufactured By, Store Ref. No.

- ﻿﻿Assign the work of sampling of the material to the authorized QA-Personnel.

- Identify all the sampling tools i.e. thief sampler and spatula through number as per Enclosure III following the numbering system as:-

  - For API ( sampling rods) : SRA-1,2

  - API ( sampling spatulas): SSA -1,2

  - For excipients ( sampling rods) -SRE -1,2

  - Excipients ( sampling spatulas)- SSE 1,2

  - For liquids (  sampling rods)-SRL 1,2

Where

A – Active, SR-Sampling rod, E-Excipient, SS- Sampling spatula, L- Liquid

- ﻿﻿Make sure that material is received from Approved Vendor.

Sampling container

- Select the sample container according to the nature of the material to be sampled on basis of
list prepared & updated.

- Use clean dry glass bottle with screw caps for liquid and transparent stoppered flask for solvents.

- Sample all light sensitive solid materials in glass bottles wrapped with foil.

- Sample all the moisture sensitive materials in airtight container.

- Use sterile tubes for sampling the material for microbial test within 24hr. of sterilization.

Sampling device

- Use stainless steel sampling devices specially designed for solid and liquid material.

- For solid materials received in drums/container use specially designed stainless steel sampling device by which material can be sampled from different level (Thief sampler).

- For liquid and solvents received in drum, use specially designed sampling device.

- For liquid material received in small container use specially designed sampling device

- For solid material received in bags use specially designed sampling device.
Clean and maintain the sampling device as per SOP NO.: BML-QA-064 on completion of sampling.

Sampling preparation

- Check the details on the REQUEST FOR TESTING RM/PM and on UNDER TEST labels' as follow.
Name of Material: Batch No: Ref. No.: Batch Size: Analyst: Test Date : Origin :

- Ensure that containers are cleaned externally, before bringing the container to be sampled to the sampling area.

- Check whether the material falls under Approved vendor list.

|  | - If the material is not from an approved vendor, then the material is not to be sampled. - Any discrepancy found should be immediately brought to the notice of the store in-charge and Q.C. Head/Q.A Manager. Damaged containers - Check the physical condition and cleanliness of raw material containers before opening for sampling. If any container found in damaged condition, intimate Raw material store in-charge to make separate MRR. - Damaged containers whose integrity is doubtful should be sampled and tested separately with a separate A.R.No. Gowning procedure - Wear fresh gown before entering into sampling area. - Cover the hands with gloves and wear a cap and mask to cover hair and nose respectively. - Cover the footwear with shoe cover. - Use separate apron, nose mask, Hand gloves, cap & shoe covers for the sampling of different material. - Mop the hands with 79% IPA and air dry before proceeding for sampling and particularly when samples are to be drawn for Microbiological testing. Sampling Before proceeding for sampling ensure: - Cleaned status for the sampling area - Availability of 'Ready for use' suitable sampling device - Removal of all previous raw material containers and used utensils. - Label the sample bag or container properly & keep it ready. Arrange the raw material containers which are to be sampled in sampling booth area. - Note down the differential pressure reading of sampling booth and other respective detail in sampling checklist. - Open the container for sampling one by one. - Observe visually for appearance of the material for any gross abnormalities like decolourization, lumps, non-characteristics odor, foreign matter and physical heterogeneity. - In case of any abnormalities observed, inform to the store in-charge and at Head/QA. Manager. - In case of all active raw materials carry out sampling from all containers in each of supplied batch for performing identification test individually for all the containers as per RM&PM Sampling Checklist. - In case all inactive materials carry out sampling from different containers in each of supplied batch according to formula n+1 e.g. for 10 containers sample from v10+1=4 containers. - Assign the sampled Container No. as follows. (for active raw material only.) e.g. If 25 containers are received then give the container no. as 1/25, to 25/25 with Reference to drum number of container sampled. - Draw the sample from different levels of the container. (i.e. From Top, Middle & bottom). The weight of the sampled quantity should be as per list prepared - Reseal the container after sampling as under and transfer it to UT area Fibre/Plastic container pack: Shrink the inside poly bag by twisting it sufficiently. Bent it & tie it using the plastic seal. - Place the lid of container & close, it tightly & seal it. Woven and Paper Bag: Close the sampling point with butter paper and seal it properly using BOPP tape. Liquid RM container: Place the lid of container & keep it tightly closed. - Collect the sample quantity as per raw material sample quantity justification ( enclosure III) as follow; For Example - Suppose 100 containers received for an AR No. and the required quantity of sample is 100gm ( for analysis and control sample) - As per sample quantity justification, select the 11 containers for sampling according to √n+1 and take the approximately 10g sample from each selected container and make a composite sample and segregate the sample for following from composite sample. - Microbiological test - Chemical test - Control sample - After the segregation of the above sample in a sterilized sample container, affix the respective label accordingly - Transfer the collected sample to QC laboratory under supervision. - If specified in the Annexure-I, take the composite sample for microbiological analysis in sterile container. - Ensure the sample containers are closed tightly and labeled properly. - Handover the samples along with MRR, Vehicle inspection checklist, sampling checklist and supplier are COA (IF received) to Q.C. department for analysis. - If the party's COA is not received, inform the Procurement Department. - Keep the retain sample and sample to be analyzed at specified storage condition as per SOP. - Mark 'UNDER TEST/ SAMPLED' LABEL on containers from which the samples are collected. Cross the original approval label of the manufacturer on all the containers. - Put signature and date the quarantine label of sampled container - Place all the containers back to their designated place after completion of the sampling. - Affix yellow colour duly filled & signed "UNDER TEST" label on each container below white label of the store. - Prepare sampling record of raw material. - Make the entry of sampling done in sampler booth log card with respective details. |
| --- | --- |

- REFERENCE AND ENCLOSURE

- ICH Q7A – Guidance of Industry, Good Manufacturing Practices ( guideline)

- SOP for cleaning of the sampling room.

- SOP for cleaning of sampling devices

- SOP for receipt , testing and release of raw materials and packing materials

ENCLOSURE

- Raw material sampling report

- Raw materials sampling log

7.0 TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality Control Head |
| --- | --- |
| QA Doc. Unit staff | Quality Control Analyst |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |

ENCLOSURE I

DOC No.: BML/QA/