- OBJECTIVE:

To provide the instructions for line clearance of the weighing room.

- SCOPE:

This SOP covers the weighing room and provided guidelines for line clearance of weighing equipment’s, environment and materials

- RESPONSIBILITIES/ACCOUNTABILITIES:

•	Operators/Production: Perform physical clearing and cleaning of the weighing areas.

•	Production pharmacist/Supervisor: Oversee the process and ensure personnel are following the SOP.

- Ensure the calibration of the weighing balance is conducted and documented.

- Ensure cleaning log of the area and weighing balance is recorded

- Ensure the temperature and relative humidity is checked and accurate

•	Quality Assurance (QA) Officer: Verify all steps have been completed satisfactorily and provide final approval.

- ABBREVIATION AND DEFINITIONS

- BMR – Batch Manufacturing Record

- QA – Quality Assurance

- PROCEDURES

- Operators shall perform an initial verification of all items on the line clearance checklist, ensuring no materials from the previous batch or product remain.

- Operators shall ensure the correct Batch Manufacturing Record (BMR) and other relevant documents for the new batch are available and approved.

- Operators must confirm the temperature and relative humidity of the weighing room are within the specified limits.

- Operators must check that the clean air grills and filters are clean and functioning correctly.

- Production shift must visually inspect the entire weighing booth, including floors, walls, and equipment, to ensure they are free of any dust, debris, or product residue from the previous batch.

- Production shift must verify that all accessories (e.g., spatulas, scoops) are clean, dry, and free of contaminants.

- Production shift must check that all equipment, such as the weighing balance, is clean and has a current, valid calibration status.

- Production shift must confirm that no raw materials, reject materials, or over-printed packing materials from the previous batch are present.

- Production shift must ensure that all equipment and area logbooks are completely filled out and updated.

- Production pharmacist must confirm that input materials for the upcoming batch have been approved and are in the correct quantities.

- The production pharmacist /shift head in-charge fills out the line clearance checklist for final approval to the Quality assurance personnel.

- Quality Assurance personnel then review the checklist, performing their own cross-checks of the documentation, area, and equipment.

- Ensure that the personnel use the approved formulation sheet always while weighing of raw materials.

- Upon satisfactory verification of all points, QA personnel sign the checklist or BMR, granting line clearance for the production to begin.

Note

- Only one product should be processed in the weighing room to prevent cross-contamination.

- Ensure the weighing balance is calibrated for accuracy before use.

- All waste bins should be cleared and cleaned after the previous batch is removed.

Materials, Equipment, Apparatus and Reagents

- Dispensing spoons,

- Dispensing scoops

- Distilled water

- BMR

- Soft brush

- REFERENCE AND ENCLOSURE

In-house Method

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | Mixing unit staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |