- OBJECTIVE:

The purpose of this SOP is to describe the procedure for the Filling Machine to ensure filling accuracy, product quality, regulatory compliance, and minimize errors.

- SCOPE:

This SOP is applicable to cover filling machine which need to be periodically calibrated

- RESPONSIBILITIES/ACCOUNTABILITIES:

Filling Operator: run checks, follow procedures and perform calibration

Production Supervisor/ Production Pharmacist: oversee and maintain good records

IPQA officer: verify and ensure compliance

- ABBREVIATION AND DEFINITIONS

SOP – Standard Operating Procedure

IPQA – in-process Quality Assurance

- PROCEDURES

- Ensure machine is clean

- Verify all safety guards are in place and check power supply

- Set parameters (volume, speed) as per batch record/specs.

- Flush the solution line for 5-10min with hot WFI

- Adjust the settings to the required volume i.e. 100ml, 500ml, 1000ml

- Press start on the filling machine and allow solution run for 5 minutes and press stop.

- Fix the pouches on the machine and press start again

- Fill the pouches with the solution and dispense it into the measuring cylinder to check the volume

- If the volume is not accurate/satisfactory, readjust the settings until the required volume.

- Record all data (date, instrument details, readings, adjustments).

- QA verifies data and accuracy.

- Apply a "CALIBRATED" label.

- If calibration fails, stop using the machine and apply an "UNDER MAINTENANCE" label.

- Report critical failures to Head QA for investigation (as per incident SOP).

Frequency: To be done for every batch

IPQA: To check the filled volume of a product every 15 minutes.

- REFERENCE AND ENCLOSURE

ENCLOSURE 1- Filling Machine Calibration Log - BML-QDG-QA-035-F01

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |

ENCLOSURE 1

Filling Machine calibration Logbook

| Stage | CORRELATED IN TERMS OF FILLED VOLUME |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Date : Time started: |  |  |  |  |  |  | Filling Specification: 97.50ml – 103.5ml – 100ml 485.0ml – 540.ml - 500ml 980.0ml – 1080.0ml - 1000ml |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Time Stopped: |  |  |  |  |  |  |
| - Adjust the volume to not less than the expected final volume. - Cross check using measuring cylinder at delivery point of the nozzles. - Check the volume of filled pouch by cutting with a stainless steel scissors and measure the content volume using a measuring cylinder - Check the volume every minutes of filling operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FILLED VOLUME FROM DELIVERED NOZZLES |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TIME | 1 | 2 |  | 3 | 4 | 5 |  |  | 6 | 7 | 8 | 9 |  | CHECKED BY |
|  | FILLED VOLUME FROM DELIVERED NOZZLES |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TIME | 10 | 11 |  | 12 | 13 | 14 |  |  | 15 | 16 | 17 | 18 |  | CHECKED BY |
|  | FILLED VOLUME FROM DELIVERED NOZZLES |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TIME | 19 | 20 | 21 |  | 22 |  | 23 |  | 24 | 25 | 26 |  | 27 | CHECKED BY |