- OBJECTIVE:

To provide written guidelines for Storage of Raw Materials and Packing Materials.

- SCOPE:

This SOP covers the process for Storage of Raw Materials and Packing Materials

- RESPONSIBILITIES/ACCOUNTABILITIES:

Storekeeper, Store Assistant, Q.A Officer, Quality Assurance Manager, Supt. Pharmacist/ Factory Manager.

- ABBREVIATION AND DEFINITIONS

FIFO: First in First out

HDPE: High Density Polyethylene

ROPP: Roll-On Pilfer Proof

Q.A: Quality Assurance

- PROCEDURES

Storage conditions as per USP:

| Storage Condition | USP |  |
| --- | --- | --- |
| Controlled Room temperature | 15°C – 30°C |

- Storage of material in the warehouse is the next step after material is successfully received and goods inward is done.

- The received material must be stored as per applicable storage condition for the material. Material shall be handled using Pallet Truck for proper handling and to prevent damage.

- Material shall be stored as per labeled claim/ storage condition defined on the container label.

- In case of material storage condition is not specified on the label, verified on the Certificate of Analysis for storage condition of the material.

- For the materials, where storage condition is not provided by manufacturer on Certificate of Analysis as well as in the certificate of analysis, list of storage condition shall be maintained as per format for storage condition list.

Following steps shall be followed to maintain the storage condition list.

- Stability data of material shall be requested from the material manufacturer and based on the long term stability condition; material storage condition shall be decided.

- In case, sufficient data are not available, pharmacopoeia can be referred to identify the material storage condition.

- In case, there is no data and reference available, material shall be stored <25°C.

- Material shall be stored in the warehouse as per defined storage condition area or room. The location within the room shall be decided based on temperature mapping study.

- Do not store material at the area within the room or storage facility, which is not covered during the mapping study. For example, material should not be stored in room walkway or corridor if the room walkway or corridor is not covered during the mapping study.

- Material shall be stored in segregated manner. Different batch of same material shall be stored on different pallets. Different material shall be stored on different pallet.

- If the raw material damaged during storage, handling and exposed to environment, information shall be given to QA for observation and disposition decision regarding the material.

- Material shall be segregated as per its status of approval. Quarantine/ under test, approved and rejected materials shall be stored separately.

- In an event where already approved materials are found damaged, an investigation shall be raised and its CAPA filled.

- Rejected material shall be stored under lock and key. Key should be available with Quality Assurance person. It shall be ensured that rejected material shall not be accidentally used or misused.

- Solvent and liquid shall be stored on Spill Pallets to ensure that if any spillage occurs, it can be easily contained and prevent spreading in entire storage area and prevent possibility of accident.

Storage of Packaging material:

- Packaging material shall be stored as per storage condition recommended by manufacturer

- Following packing material shall be stored <25°C, or as recommended by manufacturer:

- Mono-carton

- Polycarbonate Stopper

- Shipper Carton

- Nylon/HDPE

- Bottle

- Pouches

- ROPP Cap

- Measuring Cup

- Leaflets

- Stickers

- Care should be taken while storage of silica gel canister as it is very much sensitive to moisture.

- Similarly, pre-sterilized material shall be stored to ensure that sterility of material container should not be compromised.

- The printed packing materials shall be stored in lock and key under safe and secure area

- If the materials are rejected by Quality Control Dept., Label the materials as REJECTED and transfer them to Reject Materials Room

Materials, Equipment, Apparatus and Reagents

- Pallet truck

- Plastic and stainless steel pallets

- Stock Card

- File

- Note book

- REFERENCE AND ENCLOSURE

In- House Method

7.0 TRAINING INVOLVEMENT

| All storekeeper | All sectional heads |
| --- | --- |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | STORE |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion enclosures to parent SOP |