- OBJECTIVE:

To lay down the cleaning procedure for Environmental Condition of Store Area.

- SCOPE:

This SOP shall be applicable for Environmental Condition of Store Area.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Store Assistant, Store Officer, Quality Assurance Officer, Q.A. Manager, Supt. Pharmacist/ Factory Manager

- ABBREVIATION AND DEFINITIONS

## NIL

- PROCEDURES

- Make use of hygrometer to check the temperature and humidity of the material unit

- Fill the water holding cup Hygrometer at the wet bulb end daily.

- Ensure that the dry bulb is free of any dirt attached to it

- Take the reading on the dry bulb first, and then read the wet bulb.

- Calculate your depression of the wet bulb by subtracting the reading of the dry bulb from the  reading of the wet bulb

- Check the chart attached to the slate and read on the dry bulb the temperature, check for the corresponding reading of the depression for your humidity reading.

- Record your readings in the temperature and relative humidity monthly chart BML/QDG/QA/043/F01 for daily recording of the temperature and humidity.

- Report any abnormal temperature and humidity immediately to the Q.A. Manger for necessary action.

- REFERENCE AND ENCLOSURE

- Temperature and relative humidity monthly chart - BML/QDG/QA/043/F01

7.0 TRAINING INVOLVEMENT

| All storekeeper | All sectional heads |
| --- | --- |
| IPQA officer | Store head |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | STORE |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 01 | 00 | Due for review |
| 02 | 01 | For update |
| 05 | 04 | Due for review |
| 06 | 05 | - Change in sop format. - Inclusion enclosures to parent SOP |