- OBJECTIVE:

To provide a method for replacing Reverse Osmosis Membrane.

- SCOPE:

To provide written guidelines for replacing Reverse Osmosis Membrane.

3.0 RESPONSIBILITY/ACCOUNTABILITY:

Water treatment Operator/Officer, Engineer, Q.C Chemist, Microbiologist.

- ABBREVIATIONS AND DEFINITIONS:

RO – Reverse Osmosis

- PROCEDURE:

-  Close the input valve into the RO system.

- Drain all the water in the system from the sampling points.

-  Remove the membrane housing one after the other.

-  Loose the clamps from each of the membrane housing and remove one after the other.

- Remove the membrane from the housing.

- Clean the inner part of the housing with Hydrogen Peroxide solution

- Insert a new membrane into the housing ensuring it fits well.

- Cover the housing with the covers and tightly screw the clamps for firmness.

- Close all the sampling points.

-Ensure there are no leakages

- Sanitize the RO system with Hydrogen Peroxide solution.

- Check the quality of the RO Water as per SOP

- REFERENCE/ENCLOSURE:

In-house

- TRAINING INVOLVEMENT

| Chief Engineer | Engineer-Incharge, Operators/Assistants. |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | WATER TREATMENT UNIT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |