- OBJECTIVE:

To provide written guidelines for weekly cleaning of clean room area.

- SCOPE:

This SOP covers the procedure to follow for weekly Cleaning of clean room area

- RESPONSIBILITIES/ACCOUNTABILITIES:

Production Supervisor, Production Pharmacists, Quality Assurance Officer.

N.B: GENERAL: Weekly cleaning of the Clean Room Area is planned when there is production shutdown, during weekend or any other holidays.

- ABBREVIATION AND DEFINITIONS

NIL

- PROCEDURES

- De-fumigate the Clean Area if it was fumigated previously according to the SOP.

- Stop the air-handling unit. Enter the area wearing the clean area garments.

- Take out all the dispensed material from the Material Airlock & keep it in dispensing room.

- Close all the lids of the tanks.

- Wrap any opening of pipes & nozzles of filling machine with aluminium foil.

- With the spray gun spray 2.5 % Hydrogen Peroxide solution on the floor & leave it for half an hour.

- Take detergent solution in bucket & with green sponge, scrub the SS surface of Tanks and Filling machine.

- With help of sponge scrubber scrub the Walls, Doors, Glasses & ceiling of the whole area.

- Remove the return air grills of AHU & clean them.

- Then start cleaning of floor with the soft plastic bristles.

- Wear latex hand gloves while scrubbing as Hydrogen Peroxide is corrosive to skin. Also avoid contact of hydrogen Peroxide Solution with eyes.

- After finishing all cleaning, rinse the ceiling, wall, SS Surfaces with DM water. And finally wash the floor with DM Water.

- Fumigate the area after finishing all the cleaning activity as per SOP No. BML-QDG-QA-013

- Document all activity in register properly & with signature of Production Pharmacist & QA/QC.

## EQUIPMENTS & CLEANING AGENTS REQUIRED

- Plastic bristle scrubber (Soft)

- Detergent powder

- Hydrogen peroxide/sodium hypochlorite solution

- Green sponge

- Sponge scrubber

- Spray Gun

- Clean lint free dusters

- REFERENCE AND ENCLOSURE

BML-QDG-QA-013

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |