- OBJECTIVE:

To provide the method for Batch to Batch cleaning of all production equipment.

- SCOPE:

This SOP applies to all production equipment involved in the manufacturing and packaging of Large Volume Parenteral.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Operators, Production Supervisor, Production Pharmacist

- ABBREVIATION AND DEFINITIONS

NA

- PROCEDURES

Weighing Balances

- Clean the platform of the balance with a cloth to remove any spilled powders of the previous batch

- Ensure that there are no materials from the previous batch remaining

Mixing Tanks

  - Ensure that the mixing tank is empty and that the solution of the previous batch is filled

  - Allow hot Water for Injection to flow through the tank;

  - By using the hose, clean the inside walls, stirrer and covers of the mixing tank with running hot water

  - Drain off the water and fill the mixing tank with some hot processed water

  - Flush through the holding tank and filling unit for 2 minutes and drain off the water before commencing manufacturing of the next batch

Autoclave

  - Ensure that there are no pouches of the previous batch left in the autoclave

  - Ensure proper cleaning of the autoclave is done during the week

Visual Inspection, Leak Testing and Overwrapping Machine

  - Clean the Visual Inspection view table, Leak Testing and Overwrapping Machine with lint free dedicated cloth to remove any adhering dirt

  - Clean the packaging table with neat cloth and IPA

  - Ensure that there is no left over materials from the previous batch lying in the packaging area

Materials, Equipment

- Cleaning Record Sheet

- Dedicated cleaning equipment

- Rinse water supply

Documentation and Record Keeping

- Make sure all relevant documentation, including cleaning records, verification test results, and any deviations or incidents encountered during the cleaning process.

- Ensure that all entries are clear, accurate, and signed and dated by the personnel assigned the task.

- File the cleaning record appropriately and in a well-organized manner for reference purposes.

- REFERENCE AND ENCLOSURE

In-House

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |