- OBJECTIVE:

To provide written guidelines for sanitization of mixing tanks.

- SCOPE:

This SOP covers the procedure for sanitization of Mixing Tanks.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Mixing operators, Production Pharmacists, Quality Assurance Officer.

- ABBREVIATION AND DEFINITIONS

S/S- Stainless Steel

WFI- Water for Injection

Q.C- Quality Control

- PROCEDURES

FREQUENCY: Once in a week during weekly maintenance shut down.

Before commencement of production after more than 48 hours of production stoppage.

- Ensure that mixing tank is empty and all the solution of the previous product is filled.

- Add 0.5g of caustic soda to 4L of WFI in a S/S bowl and ensure it dissolves completely.

- Use a dedicated scrub soaked into the caustic soda solution to scrub in inner wall of the tank.

- Scrub the stirrer blades thoroughly including the upper part of the tank..

- After adequate scrubbing, flush the tank with WFI using the water hose for 5 minutes while the outlet valve is open.

- To 4L of water, add 0.5g of citric acid.

- Use another long brush soaked in the resulting solution to scrub every inner part of the tank as done above.

- After adequate scrubbing, flush the inner part of the tank with WFI thoroughly for 5 minutes.

- Open the inlet valve and the outlet valve and let WFI flow continuously for 10 minutes.

- Close the tank lid, and the valves of the mixing tank.

- Repeat the above procedures for each of the mixing vessels.

- Call QC personnel to take water sample for analysis.

- Once the tank passes the analysis, ensure the pass label is affixed on the tank.

- REFERENCE AND ENCLOSURE

NIL

7.0 TRAINING INVOLVEMENT`

| Production manager | Production pharmacist |
| --- | --- |
| Sectional heads | All production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |