- OBJECTIVE:

This SOP covers the procedure for cleaning and operating of computerized semi-automatic Filling Machine

- SCOPE:

To provide written guidelines for cleaning and operating the computerized Semi-Automatic Filling Machine

- RESPONSIBILITIES/ACCOUNTABILITIES:

Production Pharmacist, Production, Q.A Officer, Filling Machine Operators.

- ABBREVIATION AND DEFINITIONS

N/A

- PROCEDURES

Precaution:

- Always wear your proper overall in the Filling Room.

- Ensure that the machine is always clean and dry.

- Always mop up the floor.

Cleaning

- Put ‘ON’ the switch from the mains.

- Then switch ‘ON’ the machine.

- Press ‘START’ button on the screen.

- Select ‘English’ Language Option.

- At the bottom left hand side of the screen press ‘Cleaning Off’ button.

- Allow hot WFI to flow through the three nozzles for 5 minutes.

- Use a dedicated clean napkin soaked in 70% IPA to clean the body of the machine until clean.

Filling operation

- Calibrate the volume of the solution by pressing the ‘CAPACITY’ button at the top left hand side of the screen (after selecting the ‘English’ button).

- Select ‘1#channel set’ to set the volume of the first nozzle.

- Select from numbers 1-8 the number that corresponds to the volume to be filled and enter the number in the Group box at the top left hand corner of the screen.

- Press Enter.

- Press Enter again at the bottom left hand side of the screen.

- Repeat this for the second and third nozzles by selecting 2#channel set and 3# channel set respectively.

- Set time interval for filling by pressing the ‘INTERVAL’ button.

- Fix the bags on the nozzles.

- Then press the ‘FILLING’ button to fill the bags with solution.

- Once the bag is filled, the solution will automatically stop.

- Remove the pouch, then cork with Stoppers provided within 2 seconds.

- Put the pouches in the container.

- Allow IPQC to check volume every 30 minutes of operation.

- Change filter after each batch filling.

- At the end of each operation, do line clearance, (See SOP for Line Clearance), and document.

- Record the total number of pouches filled.

- REFERENCE AND ENCLOSURE

Manufacturer’s Manual

7.0 TRAINING INVOLVEMENT

| Production manager | Production pharmacist |
| --- | --- |
| Sectional heads | All production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |