- OBJECTIVE:

To provide written guidelines for cleaning and sanitization of Filtration unit and Filling Machine.

- SCOPE:

This SOP covers the procedure for cleaning and sanitization of Filtration and Filling Machine.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Filling Operators, Production Pharmacists, Quality Assurance Officer.

- ABBREVIATION AND DEFINITIONS

WFI- Water for Injection.

- PROCEDURES

FREQUENCY: Once in a week during weekly maintenance shut down.

- Wear the appropriate PPE as stated in the SOP.

- Pour 2g of citric acid into 5L WFI water collected in a clean bowl.

- Dismantle the filtration unit and remove the membrane filter.

- Drop all the loosen parts of the filtration units into the citric acid solution including the 8 bolts.

- Allow it to stay in the solution for 5 hours.

- Drain the water and add fresh WFI water.

- After this, remove the water and add fresh hot water for injection to rinse thoroughly.

- Re-fix the dismantled filtration units without membrane filters.

- Wipe the outer surface of filling nozzles with clean napkin.

- Pass hot WFI from the mixing tank through the filtration units, tubes and filling nozzles for 10 minutes.

- Call the Q.C personnel to take water sample from the nozzles for analysis.

- REFERENCE AND ENCLOSURE

INHOUSE

7.0 TRAINING INVOLVEMENT

| Production manager | Production pharmacist |
| --- | --- |
| Sectional heads | All production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |