- OBJECTIVE:

To provide the method for clearance of production lines between mixing room and filling room.

- SCOPE:

To provide the method for clearance of production lines between mixing room and filling room.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Production Pharmacist, Production Supervisor, Quality Assurance Officer.

- ABBREVIATION AND DEFINITIONS

WFI- Water for injection.

- PROCEDURES

GENERAL

Line clearance is done immediately after finishing production of a batch.

Procedure:

- Ensure that the mixing vessel is empty and close the valves.

- Allow hot WFI to flow from water treatment plant and rinse the mixing tank thoroughly for about five minutes using the pipe provided.

- Drain off the water from the mixing tank by opening the valve underneath the tank.

- Repeat the steps above again.

- Then transfer about 75L hot WFI into the already cleaned tank above.

- Send the 75Litres hot WFI water through the stainless steel pipe line into the filling room.

- Allow it to flow through all the nozzles of the filling machines.

- After this, send 15L of the solution to be filled from mixing room through the stainless steel pipe to the filling room.

- Allow it to flow through the nozzles of the filling machines.

- Allow the Q.A. Personnel to confirm that there are no traces of previous batch before starting another batch.

6.0 MATERIALS

WFI

- REFERENCE AND ENCLOSURE

BP, In House Method

7.0 TRAINING INVOLVEMENT

| Production manager | Production pharmacist |
| --- | --- |
| Sectional heads | All production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |