- OBJECTIVE:

To provide the necessary steps for coding pouches during batch manufacturing.

- SCOPE:

To provide written guidelines for coding of Batch details like Batch No., Manufacturing Date, expiry date on the printed PVC Pouches with Hot Plate Type Batch Coding Machine.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Batch Coding Operator, Production Pharmacist, Quality Assurance Officer.

- ABBREVIATION AND DEFINITIONS

BMR- Batch Manufacturing Record.

PVC- Poly Vinyl Chloride.

- PROCEDURES

Cleaning Procedure:

- Gently clean the equipment with clean soft cloth.

- Arrange the character of batch coding machine properly.

5.0 Operation Procedure

- Check the Production Plan and collect the BMR for which you want to code.

- Collect the required dispensed pouches from the weekly Store Keeper.

- Remove the plate of the Batch coding machine and put required letters and digits in the plate so that the print will result in the Batch No, Mfg. Date and Exp. Date in three rows.

- Tighten the screws of the letter holder assembly and put it back into the machine.

- Switch on the Batch Coding Machine and allow it to heat up to 15 minutes.

- Check the Batch Coding tape on the machine.  If there is no tape put the tape with the printing ink side of the tape downwards.

- After heating up of the machine test the printing on a reject pouch and check for the printing quality and correctness.

- If the test print is good, code five pouches and approve the coding from the Pharmacist.

- Fill the required details on the BMR, sign and countersign it from the Pharmacist.

- Continue coding at the specified place for coding.

- After finishing coding of required number of pouches, check the number of coded pouches and ensure required numbers of pouches according to BMR are coded and if there are rejects record and keep for cleaning.

- Write the number of coded pouches required for the batch.

- Transfer the coded pouches to the Filling Room.

Precautions

- Be cautious about the coding plate as it is hot enough to render burns when in contact with the body.

- Always use good characters for coding and ensure that the details are being coded properly.

- Ensure that that plate is not too hot to puncture the coding tape or pouch itself.

- Keep the area always clean and arranged in order to avoid mix up of pouches of different batches or product.

Materials, Equipment

- Coding Machine.

- REFERENCE AND ENCLOSURE

In House Method.

7.0 TRAINING INVOLVEMENT

| Production manager | Production pharmacist |
| --- | --- |
| Sectional heads | All production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |