- OBJECTIVE:

To provide the method for filling pouches.

- SCOPE:

To provide written guidelines for Pouch Filling.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Filling Operators, Supervisor, QA Officer, Production Pharmacist.

- ABBREVIATION AND DEFINITIONS

S/S- Stainless Steel.

- PROCEDURES

Precaution:

- Switch "OFF" the UV Light before entering.

- Always wear sterilized overalls before entering.

- Ensure that the room is properly cleaned.

- Ensure that sterile non-powder latex hand gloves are used.

- Ensure that solution lines and filling machines have been steamed and there is no trace of previous batch confirmed by In -Process Q.A analyst.

- Ensure that the coded bags are available and passed into filling room through Batch Coding pass box.

- Double check the coded bags before filling to avoid filling errors.

OPERATION:

- For both the membrane and pre-filter proper, the Operator should wear another pair of gloves.

- Flush the solution line again with 75L of WFI.

- Request solution preparation Unit Staff to shut water valve and open solution valve.

- Flush the Solution line with 15L of solution to be filled.

- Open the nozzle to fill the empty pouches.

- QC analyst confirms volume checks.

- Attach identification tag to the first two filled pouches for QC test.

- Drop pouches on trays provided and send to loading bay through the pass box.

- On completion of each batch, record both filled/unfilled pouches.

- Return the unfilled pouches to batch coding room.

- Do the line clearance.

- Document the process in appropriate record.

MATERIALS, EQUIPMENT

- Semi-automatic filling machine.

- S/S pipes.

- WFI- Water for Injection.

- REFERENCE AND ENCLOSURE

In House Method.

7.0 TRAINING INVOLVEMENT

| Production manager | Production pharmacist |
| --- | --- |
| Sectional heads | All production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |