- OBJECTIVE:

To provide written guidelines for cleaning of non-sterile areas.

- SCOPE:

This SOP covers the procedure for cleaning of non-sterile areas in the production environment

- RESPONSIBILITIES/ACCOUNTABILITIES:

House Keeping Supervisor, Quality Assurance Officer, Microbiologist.

- ABBREVIATION AND DEFINITIONS

NIL

- PROCEDURES

CLEANING OF THE AREA

- De-dust all the area viz. Corridors, Autoclave Floor, Production Floor, Finished goods Store, Weekly Raw Material Store, Main Raw Material Store, Pouch & Flat carton Store.

- Remove all visible particles from the walls with the aid of production area designated clean broom

- Then sweep the floor with clean broom.

- Clean all the window & Door glasses with glass cleaner liquid & soft mop.

- Prepare 2% v/v Soap solution & mop the area with it using clean mop-brush.

- Frequently wash the mop with clean water to remove the dirt collected in it.

DISINFECTION OF NON-STERILE AREA

- Prepare the disinfectant solution as per the schedule & of the specified concentration.

- Apply the disinfectant solution on the whole floor.

- Leave it like this for at least 5 minutes. Mop away the excess disinfectant solution from the floor after 5 minutes using clean water.

FREQUENCY OF CLEANING

- Clean the Production Floor, Autoclave area, Corridors at least three times in a shift.

- Clean the weekly Raw Material Store twice a shift.

- The Main Raw Material Store & Pouches & flat carton store is to be cleaned three times a week i.e. every alternate day.

- The Boiler Room, Water Treatment Area is to be cleaned twice a shift.

- The generator house & Engineering Workshop is to be cleaned once in a shift.

MAINTAINING THE CLEANLINESS OF AREA

- Ensure there is no accumulation of water, oil or grease on the floor.

- Be prompt to clean any water spillage.

- Any areas if dirty clean it at once at that time only irrespective of schedule.

SCHEDULE FOR DISINFECTANTS

| DAY | DISINFECTANT | CONCENTRATION | DILUTION METHOD |
| --- | --- | --- | --- |
| MON | DETTOL (Chloroxylenol Solution) | 4.8%w/v | Mix 200mL with 4L of Water |
| TUE | PURIT (Chlorhexidine gluconate and Cetrimide Solution) | 0.3% w/v + 3.0%w/v | Mix 200mL with 4L of Water |
| WED | DETTOL (Chloroxylenol Solution) | 4.8%w/v | Mix 200mL with 4L of Water |
| THU | PURIT(Chlorhexidine gluconate and Cetrimide Solution) | 0.3% w/v + 3.0%w/v | Mix 200mL with 4L of Water |
| FRI | DETTOL (Chloroxylenol Solution) | 4.8%w/v | Mix 200mL with 4L of Water |
| SAT | Sodium Hypochlorite | 6.25% v/v | Mix 200mL with 5L of Water |

MATERIALS

- Clean broom

- Water Pusher

- Soap Solution

- Mop

- Disinfectant

- REFERENCE AND ENCLOSURE

NIL

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |