- OBJECTIVE:

To provide the methods required in producing a batch of intravenous fluid

- SCOPE:

To provide written guidelines for preparing a batch of intravenous infusion

- RESPONSIBILITIES/ACCOUNTABILITIES:

Production pharmacist, production supervisors, foremen, operators

- ABBREVIATION AND DEFINITIONS

S/S= Stainless Steel

BMR= Batch Manufacturing Record

- PROCEDURES

PRE-PRODUCTION CHECKS

- Check for the pre-production plan and ensure that the raw materials are dispensed for the batch (follow appropriate requisition procedure)

- Check that the BMR is available and that it has been appropriately stamped and authorized

- Ensure that the AHU (air handling unit) is operational and that adequate pressure differential is maintained

- Also ensure that the temperature and relative humidity are within the specified range (22°C-25°C and 40-60% RH respectively).

MANUFACTURING

- Check for the batch number and manufacturing and expiry date(s) coding on the pouches in order to sign the rechecked column of the BMR

- Ensure the mixing tank is free from the previous product

- Allow QC to have a sample of the WFI from the collection loop

- After approval from QC resume collecting WFI for production

- Check the weight of the dispensed raw materials and ensure that they are correct. Report any discrepancies to the pharmacist in charge

- Check the master formulation method mentioned in the BMR and ensure that there are no modifications authorized

- Manufacture the batch according to the procedure given, if any

GOOD CLEAN PRACTICES IN MANUFACTURING

- Ensure that the clean area garments are worn properly and that the nose, mouth and hairs are covered.

- Wear latex hand gloves while weighing raw materials

- Do not touch the walls and railings of the mezzanine floor while wearing hand gloves, replace hand gloves if this occurs

- While adding acid or alkali for pH adjustment do so slowly and allow sufficient time to mix and stabilize

- Always keep the lid of the manhole of the mixing tank closed except when materials are being added

- Always write down the details required in the BMR immediately after the operation

- Any spillages of water and/or raw materials should be cleaned instantly

Materials, Equipment, Apparatus and Reagents

- Release certificates

- Status Labels

- BMR

- REFERENCE AND ENCLOSURE

INHOUSE

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |