- OBJECTIVE:

This SOP describes the steps for entering and exiting a Production environment with proper gowning to ensure minimal contamination.

- SCOPE:

To provide written procedures for removing street clothes, donning appropriate PPE, and maintaining hygiene throughout the process

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Individual staff entering into the production area.

- Head, production shall be responsible for implementation and compliance of the procedure

- QA Head shall be responsible for adequate training and compliance of the procedure

- ABBREVIATION AND DEFINITIONS

- % : percentage

- IPA: Isopropyl alcohol

- PPE – personal protective equipment

- PROCEDURE FOR GENERAL GOWNING

- Upon arrival at the main entrance to the change room area, the change room attendant will be on hand to provide a change of shoes BEFORE entering the main doors.

- Enter into the staff change room; sanitize your hand with 70%IPA.

- Remove personal items like watches, jewelry, and phones and keep them in dedicated lockers provided.

- Wear appropriate PPE, typically starting with a hairnet or bouffant cap, face mask, and beard cover (if applicable), followed by a Factory wears/ gown, and finally, shoe covers.

- Visually check the gowning for proper fit through the mirror provided before entering into the production areas

- Enter in the corridor through the other door of the change room and reach the concerned area.

- Handle garments carefully to avoid contamination. Avoid touching outer surfaces with PPE.

Exit

- From production areas to Change Room: Enter the change room without touching outer surfaces. Remove and discard gloves and any disposable items.

- Remove the PPE and the factory shoes

- Drop the garments in the laundry basket provided, sanitize hands with 70%IPA.

- REFERENCE AND ENCLOSURE

NIL

7.0 TRAINING INVOLVEMENT

| All staff | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All sectional Managers |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |