- Objective:

To provide written guidelines for conductivity meter.

- Scope:

This SOP covers the process of cleaning and operating conductivity meter in the chemical laboratory

3.0 Responsibility/accountability:

Laboratory Assistant, Quality Control Chemist, Quality Assurance Officer, Quality Assurance Manager, Superintendent Pharmacist/Factory Manager

4.0 Abbreviations and Definitions

cGMP – Current Good Manufacturing Practice

PPE – Personal Protective Equipment

SOP – Standard Operating Procedure

- Procedure

#### 5.1 Safety and General Precautions

- Always wear appropriate PPE (lab coat, gloves, and safety goggles).

- Handle the electrode with care to avoid breakage or damage.

- Never immerse the meter unit in any liquid.

#### 5.2 Cleaning Procedure

- Disconnect the conductivity meter from the mains power supply.

- Clean the outer surface of the meter and its electrode using cotton wool moistened with 70% isopropyl alcohol (IPA) or ethanol.

- Rinse the electrode only with distilled water to remove any residual solution.

- After rinsing, gently dry the electrode with soft, lint-free tissue.

- Do not use abrasive materials or immerse the probe in strong cleaning agents.

- Record the activity in the Conductivity meter Cleaning Logbook (BML-QDF-CL-010-F01)

#### 5.3 Operation Procedure

- Connect the conductivity meter to the mains power supply.

- Switch on the meter and allow a 15-minute warm-up period before use.

- Ensure the meter has been cleaned and calibrated.

- Prepare the test solution in a clean, dry beaker. Also prepare another clean beaker with distilled water for electrode rinsing.

- Avoid testing hot samples; allow the solution to cool to room temperature before testing.

- Dip the conductivity electrode fully into the test solution, ensuring the platinum sensor is completely immersed and no air bubbles are trapped.

- Avoid any impact or contact of the electrode with the beaker wall.

- Wait for the reading on the display screen to stabilize, then record the conductivity value.

- After measurement, rinse the electrode with distilled water and gently wipe dry with soft tissue.

- Switch off the conductivity meter and disconnect from the power supply after use.

- Record the activity in the Conductivity meter Usage Logbook (BML-QDF-CL-010-F02)

Materials, Equipment, Apparatus and Reagents

PPE, cotton wool, 70% alcohol

6.0 REFERENCE/ENCLOSURE

Equipment manual

ENCLOSURE 1- CONDUCTIVITY METER CLEANING LOGBOOK- BML-QDF-CL-011-F01

ENCLOSURE 2- CONDUCTIVITY METER USAGE LOGBOOK- BML-QDF-CL-011-F02

7.0 TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality Control Head |
| --- | --- |
| QC Analyst | Laboratory Assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE/ DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | CHEMICAL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |