- Objective:

To provide written guidelines for Cleaning and Operation of TDS Meter

- Scope:

This SOP covers the process of Cleaning and Operation of TDS Meter used in the chemical laboratory

- Responsibility/accountability:

Quality Control Chemist, Quality Assurance Officer, Quality Assurance Manager.

4.0 Abbreviations and Definitions

-cGMP – Current Good Manufacturing practice

-SOP – Standard operating Procedure

-PPE – Personal Protective Equipment

- Procedure

#### 5.1 Cleaning Procedure:

- Ensure the TDS meter is unplugged (if applicable) and turned off.

- Wipe the outer body of the instrument using a lint-free cloth slightly moistened with 70% IPA or ethanol.

- Rinse the probe (electrode) with distilled water before and after each use to avoid contamination or residue build-up.

- Dry the probe with soft, lint-free tissue or allow it to air dry.

- Record the activity in the TDS Meter Cleaning Logbook (BML-QDF-CL-018-F01).

#### 5.2 Operating Procedure:

- Confirm the instrument is clean and free from dust or residual contamination.

- Remove the protective cap from the electrode/probe.

- Turn ON the instrument using the power button.

- Immerse the probe in the sample solution, ensuring immersion does not exceed the maximum level indicated by the manufacturer.

- Gently stir the solution to ensure a uniform reading.

- Wait for the reading to stabilize. The meter compensates automatically for temperature.

- Multiply the displayed value by the correction factor (×10) as per manufacturer instructions to obtain the reading in ppm (mg/L).

- After the reading is taken, remove the probe, rinse with distilled water, and dry.

- Switch OFF the instrument and replace the protective cap securely.

- Record the activity in the TDS Meter Usage Logbook (BML-QDF-CL-018-F02).

Materials, Equipment, Apparatus and Reagents

- PPE

6.0 REFERENCE/ENCLOSURE

- In House Method

- ENCLOSURE 1- TDS METER CLEANING LOGBOOK- BML-QDF-CL-018-F01

- ENCLOSURE 2- TDS METER USAGE LOGBOOK- BML-QDF-CL-018-F02

7.0 TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality Control Head |
| --- | --- |
| QC Analyst | Laboratory Assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE/ DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | CHEMICAL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |