- Objective:

To provide written guidelines for Testing of Total Organic Carbon in Water for Injection (WFI).

- Scope:

This SOP covers the procedure for testing of Total Organic Carbon in Water for Injection (WFI).

- Responsibility/accountability:

QA Officer, Chemist, Quality Assurance Manager and Superintendent Pharmacist.

4.0 Abbreviations and Definitions

N/A

- Procedure

- Turn on the Total Organic Carbon analyzer.

- Enter the required password.

- Select the Offline mode option.

- Allow the system to warm up as per manufacturer instructions.

- Check the accessories and waste container. Empty if needed.

- Collect samples in TOC-clean, pre-rinsed containers (e.g., glass).

- Cover the container with foil or cap and avoid contamination – do not touch container interiors or cap interiors.

- Label the samples clearly.

- Open the software interface

- Choose “Measure” mode.

- Place sample via the Inlet.

- Enter Measurement settings e.g., sample IDs, batch name, and other metadata.

- Select the Clean option before starting the measurement (it will clean for 5 times) and gives the Total Inorganic Carbon Response (TIC Response)

- Start the measurement – the instrument will typically:

- Wash for few minutes

- Measure resulting Total Organic Carbon in micrograms per Litre (µg/l) and give the Average value with the Relative Standard Deviation (RSD).

- Measure the Temperature.

- Review and save results after run completion.

- Print reports if required.

- Turn off the instrument if not in use.

Materials, Equipment, Apparatus and Reagents

- PPE

- TOC Analyzer

- Glass beaker

- Foil

6.0 REFERENCE/ENCLOSURE

- In House Method

7.0 TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality Control head |
| --- | --- |
| Quality Control Analysts | Laboratory Assistants |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE/ DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | CHEMICAL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### 00 | First Issue |
| ### 01 | ### 00 | - Inclusion of enclosures format - Change to instrumentation method - Change in sop format generally |