- OBJECTIVE:

To provide written guidelines for Sampling of water for Chemical Analysis.

- SCOPE:

This SOP covers the process of Sampling of water for Chemical Analysis.

- RESPONSIBILITY/ACCOUNTABILITY:

Quality Control Chemist, Quality Assurance Manager, Superintendent Pharmacist.

4.0 ABBREVIATIONS AND DEFINITIONS:

-SOP – Standard operating Procedure

-PPE – Personal Protective Equipment

- PROCEDURE:

#### 5.1 Preparation for Sampling:

- Ensure all PPE (gloves, lab coat, etc.) are worn before sampling.

- Take a clean, dry 500 mL conical flask, and securely plug it with cotton wool.

- Carry the flask in a dedicated sampling basket to prevent contamination.

#### 5.2 Sampling Procedure:

- Proceed to the designated sampling point.

- Open the sampling valve/tap and allow the water to flow freely for 30 seconds to flush stagnant water.

- Remove the cotton plug and rinse the conical flask thoroughly with running water.

- Collect approximately 400 mL of the water sample into the flask.

- Immediately plug the flask back with the cotton wool to avoid contamination.

- Label the sample properly indicating:

  - Sampling Point

  - Date of Sampling

  - Time (optional, if required)

#### 5.3 Post-Sampling:

- Carry the sample directly to the laboratory.

- Perform the chemical analysis as per the relevant SOP and specification.

- RO/WFI water must be sampled daily from each point for chemical analysis.

Materials, Equipment, Apparatus and Reagents

- Conical flask (500ml)

- Sampling Basket

6.0 REFERENCE/ENCLOSURE:

- In-House

7.0 TRAINING INVOLVEMENT:

| Quality Assurance Manager | Quality Control head |
| --- | --- |
| Quality Control Analysts | Laboratory Assistants |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE/ DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | CHEMICAL LABORATORY |

9.0 REVISION HISTORY:

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |