- OBJECTIVE:

To provide written guidelines for cleaning and operation of water bath

- SCOPE:

This SOP covers the procedure for the cleaning and operation of Water Bath

- RESPONSIBILITY/ACCOUNTABILITY:

Quality Assurance Officer, Quality Control Chemist, Quality Assurance Manager and Superintendent Pharmacist.

4.0 ABBREVIATIONS AND DEFINITIONS:

-SOP – Standard operating Procedure

-PPE – Personal Protective Equipment

- PROCEDURE:

Cleaning procedure

- Ensure the water bath is turned off and disconnected from the power source.

- Allow the unit to cool before cleaning (to prevent injury or damage).

- Wear appropriate PPE.

- Wipe the external surfaces of the water bath using clean, dry or slightly damp soft cloth.

- Open the inner chamber or lid and clean thoroughly using a lint-free soft towel dampened with clean water.

- Do not use strong chemicals, use mild detergent or liquid soap only if needed.

- Inspect the electrical cord and check for any signs of damage or looseness.

- Ensure all surfaces are completely dry before reconnecting the water bath to the power source.

- Record the cleaning activity in the Cleaning Logbook (BML-QDF-CL-025-F01).

Operating Procedure

- Pure clean lukewarm water into the water cabinet up to ½ - 2/3 of its total height.

- Confirm the water level is adequate to submerge sample containers without overflow.

- Connect the power cord and turn on the main switch to the ‘I’ position (ON).

- Turn the thermostat to the required working temperature.

- Allow the bath to reach the set temperature and stabilize.

- Place the sample container on the water bath for a specified time given.

- Switch off power supply and disconnect the plug after use.

- Empty the water in the water cabinet after use.

- Dry the unit with a clean cloth.

- Record the activity in the usage logbook (BML-QDF-CL-025-F02).

Materials, Equipment, Apparatus and Reagents

- PPE

- REFERENCE/ENCLOSURE:

- Equipment Manual

- ENCLOSURE 1- Equipment Cleaning Logbook- BML-QDF-CL-025-F01

- ENCLOSURE 2- Equipment Usage Logbook- BML-QDF-CL-025-F02

7.0 TRAINING INVOLVEMENT:

| Quality Assurance Manager | Quality Control head |
| --- | --- |
| Quality Control Analysts | Laboratory Assistants |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE/ DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | CHEMICAL LABORATORY |

9.0 REVISION HISTORY:

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |