- OBJECTIVE:

To provide a procedure for calibration of bucket autoclave to ensure that the temperature, pressure and sterilization time for effective sterilization.

- SCOPE:

This procedure applies to the calibration of bucket autoclaves used for sterilization of culture media and laboratory equipment in the microbiology laboratory.

3.0 RESPONSIBILITY/ACCOUNTABILITY:

Laboratory Attendant, Quality Control Microbiologist, Quality control Head, Quality Assurance Manager, Superintendent Pharmacist.

4.0 ABBREVIATIONS AND DEFINITIONS:

N/A

- Procedure

- Ensure the autoclave is clean and empty

- Check water level in the autoclave

- Ensure power supply is stable

Biological Indicator Test

- Place the biological indicator containing Geobacillus stearothermophilus NCTC 10007 inside the autoclave

- Run a sterilization cycle

- After sterilization, incubate the indicator at 55-60°C for 24hours

- Observe the result

- Successful sterilization shows no color change, remains purple while a failed sterilization shows color change from purple to yellow

Chemical Indicator Test

- Place chemical indicator tape inside the autoclave

- Run sterilization cycle

- Observe the result ; indicator should change color indicating sterilization conditions were reached

- Record all readings in the calibration logbook in the format below (BML.QA.MCB.AV.001)

Frequency: Calibration should be carried out quarterly

Materials, Equipment, Apparatus and Reagents

- Chemical Strip

- Biological Indicator

- Bucket Autoclave

- REFERENCE/ENCLOSURE:

REFERENCE: EQUIPMENT MANUAL

ENCLOSURE 1- INCUBATOR CALIBRATION LOGBOOK - (BML.QA.MCB.AV.001)

- TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality control head |
| --- | --- |
| Quality Microbiology staff | Laboratory assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY:

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |