- OBJECTIVE:

- This procedure is written to serve as a written guide to carry out surface swabbing and its analysis

- It is being performed to ascertain the level of cleanliness of materials, equipment and surfaces.

- SCOPE:

- This procedure needs to be performed after the cleaning of certain materials, equipment and surface.

- It is application to production vats, storage tanks, nozzles of the filling machines, pipes, filling room’s floor etc.

- RESPONSIBILITY/ACCOUNTABILITY:

- Microbiologists are responsible for the collection and analysis of swabbed samples from cleaning equipment.

- The Quality control manager ensures the procedures outlined are followed.

4.0 ABBREVIATIONS AND DEFINITIONS

- IPA – Isopropyl Alcohol

- CFU: colony forming unit

- TSA: Tryptone Soya Agar

- SDA: Sabouraud Dextrose Agar

- PROCEDURE

- The Swabbing stick case should be labeled with the point to be swabbed.

- At the swabbing point, the seal of the swab should be detached, and the point is being rubbed with the cotton wool end of the swab aseptically, covering an area of 10 x 10 cm2 in unidirectional movements and not back and forth movements

- The swab should then be replaced back to its case and placed into the sampling box and taken to syrup microbiology lab. for analysis after appropriate line clearance is done

- Place the swab in another sterile test tube containing 10ml of water for injection

- Shake the tube gently and let it stand for 15mins

- Remove the swab from the test tube and analyze the water as per the specification (pour plate method)

- Negative control should be prepared alongside with the test

- The Swab should be streaked on plates containing set pre incubated media of tryptic soy agar, sabouraud dextrose agar and Eosin methylene blue agar aseptically in a laminar air flow chamber (streaking method).

- The plates should have been labeled accordingly.

- Tryptic soy agar and EMB are incubated inverted at 30 – 35oC for 24 – 48 hours while that of sabouraud dextrose agar is done at 20 – 25oC for 72 hours.

- The result should be recorded into the respective swabbing analysis record sheet Guide i.e Infusion equipment swab analysis record sheet BML/QDG/MC/007/G01 andSyrup equipment Swab analysis record sheetBML/QDG/MC/007/G02

READING AND INTERPRETATION OF RESULTS:

- The growth on TSA/NA and SDA should not exceed limit of 10 cfu/100cm2if any and there must be absence of pathogens.

REPORTING:

- However, the total viable count for the floor if any should be not exceeding the limit of 50 cfu/100cm2, fungi count should not be more than 10cfu/100cm2

MATERIALS AND EQUIPMENT:

- Sterile Petri-dishes

- Media (Casein digest soya bean broth, Casein digest soya agar, sabouroud dextrose agar, MacConkey broth, MacConkey agar)

- Sterile Swab Stick

- Incubator

- Laminar flow cabinet

- Bunsen burner

- 70% IPA

- Cotton wool

- Adhesive tape

Safety, Health and Environment Precautions

- Wear appropriate PPE

- REFERENCE/ENCLOSURE

- Infusion Equipment Swab Analysis Record Sheet - BML/QDG/MC/007/G01

- Syrup Equipment Swab Analysis Record Sheet - BML/QDG/MC/007/G02

- TRAINING INVOLVEMENT

| Quality Control Head | Quality control microbiologist, Laboratory assistant |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORIES |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |