- OBJECTIVE:

To provide written guidelines for cleaning and Operation of analytical balance in syrup chemical laboratory.

- SCOPE:

This SOP covers the process of cleaning and Operation of analytical balance used in

The chemical laboratory

3.0 RESPONSIBILITY/ACCOUNTABILITY:

- The Chemical analyst shall carry out calibration procedures using standard weights.

- The Quality Control Head reviews the results as against the specifications.

- Users shall strictly follow this procedure to achieve safe and efficient use of equipment

- The user shall ensure the equipment is kept clean and reports any malfunctioning.

- The Quality Control Head ensures scheduled maintenance is carried out.

4.0 ABBREVIATIONS AND DEFINITIONS

- Calibration: to mark of gauge an instrument with a standard scale of readings

- Analytical balance: An instrument used in the high accuracy weighing of materials

- PROCEDURE

Safety Precaution

- Ensure the analytical balance is switched OFF before cleaning.

- Check the main power source and ensure it is OFF

- Do not MOVE or change the position of the analytical balance while cleaning

Cleaning Procedure

- Open the side glass and clean the top load and inside cubicle with a dry lint cloth

- Clean the top glass and side glass with dry cloth.

- Re-clean the inside cubicle and top load with ethanol moist cloth and wipe dry after cleaning

Operation Procedure

- Switch on the main power source and the UPS attached to the balance

- Switch on the balance from the adaptor power unit and press the T button on the balance panel to power it on

- Allow the analytical balance to warm up for 15 minutes

- Calibrate the analytical balance by following the procedure in the balance calibration BML-QDG-QA-032.

- When weighing do not weigh nothing more than 150g on the analytical balance

- Place materials to be weighed on a weighing dish when weighing

- Log in all activities in the user’s logbook (LGB/QDS/CL/004/01)

| Date | Used for | Start time | End time | Total time use | Status | Operator sign | Checked by | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- To tare the balance, press the TARE (‘T’) button

- After using the analytical balance ensure its clean and not soiled by any chemicals or materials

Materials, Equipment, Apparatus and Reagents

- PPE

- Weighing dish

- Spatula

Safety, Health and Environment Precautions

- Wear appropriate PPE

- REFERENCE/ENCLOSURE

- Mettler analytical balance instruction manual.

- TRAINING INVOLVEMENT

| Quality Control Head | Chemical Analyst, Laboratory assistance |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |