- OBJECTIVE:

To provide written guidelines for cleaning and operation of UV Spectrophotometer.

- SCOPE:

This SOP covers the process of cleaning and operation of UV Spectrophotometer.

- RESPONSIBILITY/ACCOUNTABILITY:

- The Chemical analyst shall safely and properly handle the equipment.

- The user reports any deviation or equipment malfunction.

- The equipment user must ensure that the equipment is always kept balanced and clean.

- The Quality control Manager ensures external calibration is carried out as scheduled

4.0 ABBREVIATIONS AND DEFINITIONS:

- Absorbance: The amount of light energy being absorbed by the species of a chemical substance in a solution.

- Wavelength: The distance between repeating units of a propagating wave of a given frequency designated by λ.

- PROCEDURE

CLEANING PROCEDURE:

- Apart from daily routine cleaning, the UV-Vis Spectrophotometer must be cleaned once a week.

- Open and clean the Cuvette holder.

- Clean the outer part of the apparatus.

- Clean the Apparatus from its original position clean the base and then replace

OPERATION

Initial Set-up

- Connect the instrument to power supply and allow the microcomputer to conduct self-check.

- The Spectrophotometer will check the position of its light sources, mirrors and second order suppression filters after every power on cycle. It will take some time & also lamp will take some time to stabilize. So allow the machine to stabilize for 30 minutes.

Measuring sample

- Use the wavelength knob to change the wavelength setting of the instrument, and select the desired wavelength with the scale of the wavelength window.

- Select a colorimetric plate. Place the reference/test sample onto the sample holder. Adjust the position of the sample with the sample holder level.

- Zero the instrument using the TARE Button after the test wavelength has been changed or instrument has been in use for a period of time.

- Select the required display mode using the T/A/C/F mode keypad to read the direct concentration reading and concentration factor.

- When the instrument in the T, A or C display mode, connect it with a serial printer via the RS232C port. Press the Enter/Print to print the displayed data.

- Record the usage activities in the user logbook LGB/QCS/CL/010/01 for the UV Spectrophotometer appropriately.

UV SpectrophotometerUser Logbook LGB/QCS/CL/010/01

| Date | Used for | Start time | End time | Total time use | Status | Operator sign | Checked by | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note: Put in the reference sample before direct concentration reading setting.

Materials, Equipment, Apparatus and Reagents

- PPE

Safety, Health and Environment Precautions

- Wear appropriate PPE

- REFERENCE/ENCLOSURE

- UV spectrophotometer instruction manual.

- TRAINING INVOLVEMENT

| Quality Control Head | Chemical Analyst |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |