- OBJECTIVE:

To provide the procedure for cleaning of Microbiology Laboratory.

- SCOPE:

To provide written guidelines for cleaning of Microbiology Laboratory.

- RESPONSIBILITY/ACCOUNTABILITY:

Microbiologist/Assistant shall ensure the laboratory is cleaned in the morning and always 	maintained cleaned at all times.

4.0 ABBREVIATIONS AND DEFINITIONS

PPE: Personal Protective Equipment.

- PROCEDURE

Precautions

- Perform the area cleaning and sanitation when there is no activity.

- Handle the disinfectant carefully to avoid contact with skin and eyes. Use only validated disinfectants.

- Do not use expired disinfectant solutions.

- Rotate the disinfectants use to avoid the development of resistance by microorganisms.

- Use separate lint free mop and fresh solution for cleaning of different area to avoid contamination of mop and solution.

- Use only dedicated buckets/containers for preparation and filtration for different disinfectants.

Preparation for a disinfectant solution

- Take the required amount of disinfectants and add purified water to make up desired final volume.

- Prepare the volume of disinfectant according to the use.

Cleaning and sanitization

- The cleaner should ensure that no work is in progress while cleaning

- Start cleaning in the following sequence with lint free mop soaked with 70% v/v filtered IPA.

- Outer surface of equipment

- Cabinet and accessories

- View glass windows and doors

- Empty waste bin (replace the plastic liner daily)

- Mop the floor with validated disinfectant with non-fiber shedding mop as per the following sequence as mentioned below.

- Inoculating room

- Preparatory room

- Incubation room

- Change room

- Instrument room

- Inspect and ensure the cleanliness of the area and record.

- Frequency: three times a day, beginning and at the end of working hours or whenever required.

- Inspect and ensure the cleanliness of the area and record.

- The housekeeper then fills the daily cleaning chart form BML-QDG-QA-006-F01

Materials, Equipment, Apparatus and Reagents

- Deionized Water

- Soft Cloths

- Mops

- Buckets

- Soap

- Hypochlorite

- 70% IPA

- REFERENCE/ENCLOSURE

- Daily cleaning chart form BML-QDG-QA-006-F01

- TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality control head |
| --- | --- |
| Quality Microbiology staff | Laboratory assistance |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |