- OBJECTIVE:

To provide the procedure for cleaning of work bench in microbiology Laboratory

- SCOPE:

To provide written guidelines for cleaning of workbench in microbiology Laboratory.

3.0 RESPONSIBILITY/ACCOUNTABILITY:

Microbiologist/Assistant shall ensure the laboratory is cleaned in the morning and 	always maintained cleaned.

4.0 ABBREVIATIONS AND DEFINITIONS

N/A

- PROCEDURE

- Ensure that all cleaning equipment is properly washed and cleaned i.e. soft cloth and bowls.

- Always use appropriate protective wear.

- Remove loose items from the laboratory workstation. Beakers, test tubes, pipettes etc. should be relocated and washed appropriately.

- Prepare 70% IPA in a clean bowl and dip a soft cloth in the mixture and wipe the workbench surface, corners, edges and undersides thoroughly.

- Use a wire brush or other device to remove some residue.

- Caked-on material such as solidified agar or other gelatin-like products can be removed by boiling purified water in the equipment.

- An IPA rinse is useful to sterilize laboratory equipment that requires all microorganisms to be removed before use.

- Wash and dry all the cleaning equipment.

Materials, Equipment, Apparatus and Reagents

- Deionized Water

- Soft Cloths

- Bowls

- Mild Soap

- Cotton wool

- 70% IPA

- REFERENCE/ENCLOSURE

- Nil

- TRAINING INVOLVEMENT

| Quality Control Head | Chemical Analyst, Laboratory assistance |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |