- Purpose

- This SOP outlines the procedures for the microbiological analysis of PET bottles, ROPP caps, and measuring cups used in pharmaceutical manufacturing to ensure they meet the required quality and safety standards.

- Scope:

- This procedure applies to all PET bottles, ROPP caps, and measuring cups used in the manufacturing and packaging of pharmaceutical products.

- Responsibilities:

- Microbiologist:  Perform the microbiological analysis as per the SOP.

- Quality Assurance Manager: Ensure compliance with the SOP and review test results.

- Definitions:

- CFU: Colony-Forming Unit

- ROPP Cap: Roll-On Pilfer Proof Cap

- Materials and Equipment:

- Sterile swabs

- Sterile containers

- Nutrient agar plates

- Incubator (37°C ± 2°C)

- Autoclave

- Sterile diluents (e.g., sterile saline)

- Pipettes and pipette tips

- Gloves and protective clothing

- Procedure:

  - Sample Collection:

- Ensure all personnel wear appropriate PPE before entering the sampling booth.

- Ensure the sampling booth is clean and free from materials concerned to previous sampling (SOP for the operation of sampling booth BML-QA-064)

- The sample should be taken randomly according to the sampling plan of packaging materials SOP BML-QA-SYP-O34

- Label samples with relevant information (e.g., material name and date of collection).

- The collected sample should be taken to the laboratory and analyzed accordingly.

- Microbial Enumeration Test

  - Determination of Total Aerobic Microbial count (TAMC) and Total Yeast and Molds count (TYMC) by using pour plate method.

- Testing must be performed under Laminar Air Flow

- Arrange all the required material inside the Laminar Air Flow before starting the test.

- Media preparation:

- Suitable media (soya bean digest broth, Eosin methylene blue agar, MacConkey broth, soya bean digest agar and Sabouraud Dextrose Agar) are prepared as per SOP BML-MC-SYP-011

- The media is allowed to cool to about 40°C – 45°C (maximum up to 55°C), but don’t let it solidify.

- Note: If the media is prepared already and solidified, melt it by placing it over a water bath or other heat source.

  - Arrangement of glass wares inside the laminar air flow

- Arrange sterile Petri plates, sterile MacCartney bottles and sterile test tubes.

- Label the test tubes and petri plates at the edge of the bottom of the plate with the material name, batch number, analysis date, analysis completion date.

  - Sample preparation:

- Pour 50mls of sterile water for injection into each bottle to be analyzed and allow to stay for 10minutes.

- In a sterile conical flask pour 100ml of water for injection and put 10 measuring cups/ROPP caps, shake vigorously and allow to stay for 10minutes.

- Aseptically Pipette 1ml of the sample in sterile Petri dishes.

- Method used

  - Pour-plate method: for isolating and counting the viable microorganisms present in a liquid sample, which is added along with or before molten agar medium prior to its solidification.

    - Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method.

- Slightly open the lid of the petri plate and dispense 1 ml of sample in the center of the Petri plate using a sterile calibrated pipette.

- Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC)

- Pour about 15 ml of the sterilized molten media at the appropriate temperature above the sample.

- Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape.

- Allow the media to completely solidify.

- Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium.

- Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours

  - Test for Absence of Pathogen

Test for E. coli:

- Prepare the sample dilution as described in “determination of TAMC and TYMC”

- Transfer 1ml of the sample into a sterile MacCartney bottle containing 9ml of sterile MacConkey broth.

- Incubate the MacConkey broth bottle at 300C-350C for 24 hours.

- After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate.

- Incubate plates at 30°C to 35°C for 48 hours.

- For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar.

- Observe and record the presence or absence of specified microorganisms.

- Results Interpretation:

- Compare the microbial counts with acceptance criteria specified in the relevant pharmacopeia or internal standards.

- Identify and record any specified microorganisms detected.

- Documentation:

- Record all observations, test results, and any deviations in the appropriate logbooks

- Ensure all documentation is signed and dated by the person performing the test and reviewed by the Quality Assurance Manager.

- Safety:

- Follow all laboratory safety protocols, including the use of personal protective equipment (PPE).

- Dispose of all biological waste in accordance with local regulations and company policies.

- Training:

- Ensure all personnel involved in the microbiological analysis are trained in this SOP and relevant microbiological techniques.

- References:

- USP <61> Microbiological Examination of Non-Sterile Products: Microbial Enumeration Tests

- USP <62> Microbiological Examination of Non-Sterile Products: Tests for Specified Microorganisms

- In house

- REPORTING:

- The results should be recorded in the microbiological analysis of raw material record.

- The final results should be reported to the Quality Control Manager.

- Training Requirements

- On relevant SOP’s

- On good cleaning practices

- On cGMP

- On good documentation practice.

- Distribution and Documentation control

- Microbiology laboratory: To ensure compliance

- controlled copy: To be displayed in the micro laboratory

- Master copy: To be kept with the Quality Assurance Manager