1.0 OBJECTIVE

To provide the necessary steps for the operation of batch coding machine and cleaning.

2.0 SCOPE

This SOP will be applicable for batch coding machines and cleaning in the packing area of production.

3.0 RESPONSIBILITY/ACCOUNTABILITY

- IPQC personnel are responsible for ensuring that this SOP is followed for the operation.

- The operator will ensure this procedure is strictly followed.

- The production supervisor should monitor and give the necessary directives for the operation.

- Production pharmacists shall ensure the operation is carried out and it’s effective

4.0 DEFINITIONS AND ABBREVIATIONS

Mfg Date - Manufacturing date

Exp Date - Expiry date

BMR - Batch Manufacturing Record

IPA - Iso propyl alcohol

QA - Quality Assurance

SOP - Standard Operating Procedure

5.0 PROCEDURE

Cleaning Procedure:

- Gently clean the batch coding machine with lint wipe moistened with IPA.

- Clean the conveyor belt with napkin moistened with IPA.

Operation

- Ensure the cleanliness of the printing machine, its part and coding area.

- Ensure that the cartridge ink for coding is properly fixed and the sensor in place.

- Collect the required packaging materials from the store keeper.

- Before the commencement of operation, the operator should get the line clearance from Q.A officer by writing the status label with product name and batch details.

- Check the BMR to get the batch details that will be coded on the packaging materials.

- Set the code as per BMR details on the cartridge such that the print will result in Batch no., Mfg. Date and Exp. Date in three rows.

- Run the batch coding machine and check the print by loading the conveyor with a plain paper until the desired print is achieved (for the vertically mounted batch coding machine head) or by passing the sensor with hand (for horizontally mounted coding head).

- Run samples of monocarton/bottle label.

- When the overprinting of the batch details is satisfactory, get the signature from the production supervisor or production pharmacist and QA officer on the overprinted cartons/labels.

- Attach the sample of carton to Batch Packaging Record in the BMR for reference.

- Continue coding and complete the batch.

- Collect the already coded cartons in a duly labeled container or carton with batch details and store in the designated area provided.

- Record the batch details in the “Batch coding” record Logbook LGB/PDS/PR/007/01

| S/N | PRODUCT | B/N | MFD | EXD | QTY CODED | WASTE QTY | DONE BY | CHECK BY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- At the end of coding activities, ensure required numbers of packaging materials according to BMR are coded and if there are rejects, record.

- Keep the area always clean and arranged in order to avoid mix up of cartons/labels of different batches or product.

SAFETY, HEALTH AND ENVIRONMENT PRECAUTIONS

- Do not remove the batch coding printer’s plug without first switching off the mains

- REFERENCE/ENCLOSURE

- Nil

7.0 TRAINING INVOLVEMENT

| Production Manager | IPQC |
| --- | --- |
| Production Pharmacist | Operators |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | CODING ROOM |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 01 | ### 00 | To Update |
| ### 02 | ### 01 | - Inclusion of enclosures format - Change in sop format generally |