- Objective:

-    The objective of this SOP is to describe the proper method and procedure for daily sanitization of non Sterile Areas

- Scope:

- This SOP provides written guideline to follow by Personnel for effective cleaning and sanitization of non sterile areas

3.0 Responsibility/accountability:

-  Cleaners, Production Supervisor, Quality Assurance Officer, Production Pharmacist, Production Manager, Quality Assurance Manager and Superintendent Pharmacist

4.0 Abbreviations and Definitions

N/A

- Procedure

Cleaning of the Area

- With a clean neat broom brush remove all dust, dirt and particles in the Autoclave floor area, Boiler room, Water treatment area, Production floor, Packaging area, Finished goods store, Raw materials Store, Packaging materials Store and Reception area.

- Remove any cop-webs with the help of the long brush

- Clean all the windows, doors and frames with glass cleaner and  soft cotton cloth

- Prepare a 5%V/V liquid detergent solution and mop the area with it using clean mop-brush

- Frequently wash the mop-brush with clean water to remove all the trapped dirt’s collected in it

- Extra care to be taken when cleaning the edges and joints of walls with the floor so as not to stain the walls

Sanitization of the Area

- Prepare the disinfectant solution as per SOP for cleaning and sanitization of the factory floor and Drains. SOP-BML-QA-007-00

- Sprinkle the disinfectant solution all over the floor and allow to stand for 5 minutes before mopping it over with the mop-brush

- Frequency of cleaning is every three hours during production and once daily when not in production

- The raw materials, packaging and finished goods store areas are to be cleaning and moped once daily

- The boiler room area, compressor room area and distribution board area are to be cleaned every alternate day. i.e. three times weekly

- The generator house is to be cleaned once a week, but any spilled fuel or oil must be cleaned immediately

NOTE: ANY AREA IF DIRTY SHOULD BE CLEANED IMMEDIATELY IRRESPECTIVE OF THE SCHEDULE

Materials, Equipment, Apparatus and Reagents

- Liquid Detergent/Soap

- Mop-brush

- Broom brush

- Dustpan

- Disinfectant

- Hand Towel

- Hand dryer

Safety, Health and Environment Precautions

- Ensure there is no accumulation of water, oil or grease on the floor.

- Be prompt to clean any water spillage

- Wear appropriate PPE

- REFERENCE/ENCLOSURE

In-House

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 02 | ### 01 | ### July 2022 |  |
| ### 03 | ### 02 | ### July 2024 | - Inclusion of enclosures format - Change in sop format generally |