- Objective:

- To provide steps to be employed in the fumigation of the clean area.

- Scope:

-	To provide written guidelines for effective fumigation of the clean area.

3.0 Responsibility/accountability:

- Clean Area Supervisor, Microbiologist, Quality Assurance

4.0 Abbreviations and Definitions

N/A

- Procedure

Addendum

- Fumigation is done when the bio-burden of the area is confirmed high by the QC

Pre-fumigation:

- Preparation for the operation of fumigation.

- Ensure the area is cleaned thoroughly before fumigation.

- Close all the lids of manufacturing tanks properly. Open lines should be well closed with Aluminum foil including the M/C nozzles.

- Switch off the air-conditioner. Close all the openings like pouch transfer duct, pass boxes or any other opening.

Fumigation

- Wear clean room garments, latex hand gloves and nose mask.

- Ensure that there are no persons working in the clean area.

- Take 10 stainless steel bowls and weigh 10 sachets of potassium permanganate of 6gm each.

- Place the stainless steel bowls at different location of clean area;

- Two in change room

- Four in filling room

- Four in mixing room

- One in weighing room

- Two in Batch Coding room

- Two in clean Area corridor

- REFERENCE/ENCLOSURE

British Pharmacopeia

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 02 | ### 01 | ### July 2022 |  |
| ### 03 | ### 02 | ### July 2024 | - Inclusion of enclosures format - Change in sop format generally |