- Objective:

To provide written guidelines for effective fumigation of the non-clean area(s).

- Scope:

This SOP provides written guideline to follow for effective fumigation of the non-clean area(s).

3.0 Responsibility/accountability:

Microbiologist, Clean Area Supervisor, Q A Manager

4.0 Abbreviations and Definitions

N/A

- Procedure

Preparation for the Operation of Fumigation

  - Ensure the area is cleaned thoroughly before fumigation

  - Ensure all openings of equipment materials in the area are well covered.

  - Switch off the air extractor and other electronic gadgets.

Procedure:

  - Wear clean room garments, latex hand gloves and nose mask

  - Ensure that there are no persons working in the non-clean area

  - Take the required stainless steel bowls and weigh 10g of potassium permanganate of 6gm each.

  - Place the stainless steel bowls at different location of non-clean area		-	                           -            Four in Store

-	Two in Quarantine

-	Two in Packaging

- Measure and pour 18mls of Formaldehyde to each of the containers.

- With caution, start adding the potassium permanganate in each of the bowls, starting from the one end so that after addition of the last sachet, you can come out of the room without encountering fumes.

- Paste label at the entrances stating “AREA FUMIGATED, DO NOT ENTER”

- Leave the area fumigated as such for at least 24 hours.

- Fumigation of this area is done fortnightly or when there is shut down of the production activity.

SAFETY, HEALTH AND ENVIRONMENTAL PRECAUTIONS:

- Always wear protective coverings and hand gloves while handling formaldehyde.

- After addition of potassium permanganate to formaldehyde, there happens effervescence. Be prompt to leave the particular place before this.

- REFERENCE/ENCLOSURE

In House Method

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 02 | ### 01 | ### July 2022 | To update |
| ### 03 | ### 02 | ### July 2024 | - Inclusion of enclosures format - Change in sop format generally - To update |