- Objective:

To provide written guidelines for cleaning and operation of sampling dispensing booth

- Scope:

This SOP covers the procedure to follow for cleaning and operation of sampling dispensing booth

- Responsibility/accountability:

Production personnel, Production Supervisor, Production Pharmacists, Quality Assurance Officer, Quality Assurance Manager, Superintendent Pharmacist/Factory Manager.

4.0 Abbreviations and Definitions

N/A

- Procedure

## CLEANING

- Ensure you wear PPE

- Remove the cobwebs with the help of the long broom.

- Clean the Door glasses with glass cleaner liquid & soft mop.

- Prepare 2% v/v Soap solution & mop the area with it using clean mop-brush. Frequently wash the mop with clean water to remove the dirt collected in it.

- Take extra effort to clean the joints between wall & floor as it is likely to accumulate more dirt.

SANITIZATION OF THE AREA

  - Prepare the disinfectant solution of the specified concentration.

  - Apply 5%v/v sodium hypochlorite solution to disinfect the whole floor.

  - Leave it like this for at least 10 minutes. Mop away the disinfectant solution from the floor after 10 minutes to render the floor dry.

  - Use 50% Ethanol and clean towel to clean the Lamina Air Flow Machine

FREQUENCY OF CLEANING: Daily

OPERATION

- Switch ‘ON’ the mains

- Switch ‘ON’ the light.

- Start blower at slower speed one hour before the sampling.

- Note the reading of differential pressure daily.

- Always open material for sampling when the system id ‘ON’

- Sample the material as per material sampling SOP: BML-CL-SYP-014

- Don’t take sampling outside the booth, but collect in a closed container in booth itself, to avoid any spillage outside the booth.

- After sampling every raw material, note the manometer reading.

- Use clean sampling device for different material.

- Close the containers of sampled material under laminar flow unit, before taking it out

- Put ‘off’ the system after use.

Materials, Equipment, Apparatus and Reagents

- PPE

- Clean towel

- latex gloves

- Sodium hypochlorite

- Mops

- Sweeping brush

- Plastic Packer

- Water

- REFERENCE/ENCLOSURE

IN-HOUSE

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 00 | ### Nil | ### Nil | First |
| ### 01 | ### 00 | ### Jan 2024 | - Inclusion of enclosures format - Change in sop format generally |