- Objective:

To lay down a procedure for Cleaning and maintenance of sampling devices.

- Scope:

This standard operating procedure is applicable for cleaning and maintenance of sampling devices after sampling drug products and different raw materials.

- Responsibility/accountability:

QA Officer, Chemist, Microbiologist, Quality Assurance manager, and Superintendent Pharmacist

4.0 Abbreviations and Definitions

N/A

- Procedure

- After sampling, keep the sampling device (sampling beakers, sampling rods, sampling spatulas, and spoons) in a polybag labeled as “TO BE CLEANED” and transfer it to the washing area.

- Clean the total surface of sampling devices with running potable water.

- Finally rinse with purified water.

- Allow to drain the water from the sampling device and wipe with clean wet (with 70% IPA) lint-free cloth. Wipe the sampling device with a Clean, dry lint-free cloth.

- Check visually for the cleaning of the device and re-clean if required/found dirty.

- After sampling each batch, the above cleaning procedure for sampling devices shall be followed.

- The cleaned sampling device shall be used within seven days. If not, re-cleaning shall be done.

- After cleaning ‘Cleaned Label’ shall be affixed (refer to Sop For Label Control – BML–QA-097.00)

- Put the clean and dried equipment in a poly-bag and keep it at appropriate place.

Sampling pipettes (Glass)

- Clean the equipment with running tap water followed

- Finally rinse with purified water.

- Check the cleanliness physically and clean again if it is necessary.

- Keep the washed tubes for draining on the stand meant for the same.

- After the tubes are dried the ends are covered with poly-bags.

Sampling rods (Stainless Steel)

- Clean the things with running tap water

- Finally rinse with purified water.

- Check the cleanliness physically and clean again if required

- Wrap the clean and dried equipment in a poly-bag & keep it in appropriate place.

- After cleaning record to be maintained

- After cleaning sampler can be used within 48 hours.

Materials, Equipment and Apparatus

- sampling beakers,

- sampling rods,

- sampling spatulas,

- spoons/Scoops

- purified water

- 70% IPA

- Lint-free cloth.

- REFERENCE/ENCLOSURE

Pharmaguideline

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 00 | ### Nil | ### Nil | First |
| ### 01 | ### 00 | ### Jan 2024 | - Inclusion of enclosures format - Change in sop format generally |