- Objective:

To provide written guidelines for line clearance for product to product cleaning of all

Equipments.

- Scope:

This SOP describes the procedure for line clearance for product to product cleaning of all

Equipments.

3.0 Responsibility/accountability:

Production Supervisors, Production Pharmacists, Quality Assurance Manager, and Superintendent Pharmacist.

4.0 Abbreviations and Definitions

N/A

- Procedure

WEIGHING BALANCES

- Clean the plat form of the balance with cloth remove spilled powders of previous batch clean the display board, balance plat form with lint free duster moistened with disinfectant.

- Clean the dispensing spoons, scoops with hot distilled water and Soft brush. Rinse three times with hot distilled water.

MIXING TANK

- (Note: product-to-Product cleaning is required only when the previous product contains   any ingredient, which is not present in the next product   to be continued)

- Ensure that the mixing vessel is empty and close down the valves

- Allow the distilled water to flow into the empty mixing tank and collect the distilled water in a bucket and rinse the inside walls of the mixing tank thoroughly and drain the water from mixing tank.

- Ensure that the stirrer and mixing tank covers are cleaned properly.

- Drain off the water from mixing tank.

- Repeat the steps 1.2  - 1.4.

## FILTRATION AND FILLING UNIT

- Dismantle the filtration unit and remove the filters. Clean the filtration unit with hot distilled water and fix it again without membrane filters.

- Fill the mixing tank with some qty of hot distilled water and flush hot distilled water through the filtration unit and filling nozzles continuously for 5 minutes.

- Dismantle the filter holding unit and fix the membrane filters and pass hot distilled water continuously for 2 minutes and drain the water from the mixing tank and run the transfer pump of filtration unit continuously to remove all the water in the pipe line till no water is coming from the nozzles.

## AUTOCLAVE

- Ensure that there are no pouches of previous batch left in the autoclave.

- Ensure proper cleaning of autoclave is done during the weekly shut down maintenance.

## VISUAL INSPECTION AND PACKING LINE

- Clean the visual inspection view table with cloth and remove any adhering dirt.

- Clean the packaging table with cloth.

- Ensure that no left over of previous product materials are lying in the packaging area.

Ensure Proper Line clearance is taken for all equipment before starting of the next product

- REFERENCE/ENCLOSURE

In House Method

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 02 | ### 01 | ### July 2022 | To update |
| ### 03 | ### 02 | ### July 2024 | - Inclusion of enclosures format - Change in sop format generally - To update |