- Objective:

To provide written guidelines for cleaning and sanitization of Microbiological glassware.

- Scope:

This SOP covers the process of cleaning sanitization of Microbiological glassware used in the Microbiology Laboratory

3.0 Responsibility/accountability:

Laboratory Attendant, Quality Control Microbiologist, Quality Assurance Manager, Superintendent Pharmacist.

4.0 Abbreviations and Definitions

N/A

- Procedure

- Clean the glassware with plastic brushes and liquid detergent solution.

- After cleaning rinse the glassware with water until no trace of detergent is observed

- Do final rinse with water for injection

- Arrange the cleaned glassware on the glassware rack and ensure it is not in position for it to fall off.

- Before use, disinfect with 70% IPA and put it inside oven for dry heat sterilization at 1100c to 1250c.

- Ensure the laboratory sink is Clean after cleaning the glassware.

MATERIALS, EQUIPMENT, APPARATUS AND REAGENTS:

- PPE

- Hand gloves

- 70% IPA

- Nose Mask

- Plastic brush

- oven

6.0 REFERENCE/ENCLOSURE

IN-HOUSE

- TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality control head |
| --- | --- |
| Quality Microbiology staff | Laboratory assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |