- Objective:

To provide written guidelines for cleaning and Operation of Lamina Air Flow to maintain a sterile environment.

- Scope:

This SOP covers the process of cleaning and Operation of Lamina Air Flow

- Responsibility/accountability:

Laboratory Attendant, Quality Control Microbiologist, Quality Assurance Officer, Quality Assurance Manager, Superintendent Pharmacist.

4.0 Abbreviations and Definitions

IPA: Isopropyl Alcohol

- Procedure

Safety Precaution

- Ensure the Lamina Air Flow is switched OFF before cleaning.

- Check the main power source and ensure it is OFF

- Do not MOVE or change the position of the UV light and the light bulb

Cleaning Procedure

- Open the side glass and clean the bench inside with cotton wool and 70%IPA

- Document the activity in the Lamina Air Flow Cleaning Logbook (BML-QDF-MC-008-F01).

Operation Procedure

- Connect to the main electricity supply.

- Put on the ultraviolet ray florescent by pressing down the UV knob.

- For disinfection of the chamber, 30mins to 1hour of exposure to UV light is adequate.

- Switch off the UV lamp after the disinfection time.

- Switch on the normal light and airflow. The airflow supplies sterile air, and can even be put on during sterilization.

- It is advisable not to put on the normal light to prevent possible photo reactivation.

- Carry on the operation in the hood as fast as possible.

- Switch off the machine after the operation and disconnect from the main electrical supply.

- Clean the machine after the operation.

- Record the usage in the Lamina Air Flow Usage Logbook (LGB-QDF-MC-008).

Materials, Equipment, Apparatus and Reagents

- PPE

- Cotton wool

- 70% IPA

- REFERENCE/ENCLOSURE

REFERENCE: IN HOUSE, EQUIPMENT MANUAL

ENCLOSURE 1- LAMINA AIR FLOW CLEANING LOGBOOK- (BML-QDF-MC-008-F01).

ENCLOSURE 2- LAMINA AIR FLOW USAGE LOGBOOK- (LGB-QDF-MC-008).

- TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality control head |
| --- | --- |
| Quality Microbiology staff | Laboratory assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |