- Objective:

To provide written guidelines for Limulus Amebocyte Lysate LAL (test) i.e. detecting of endotoxin in a product.

- Scope:

This SOP covers the process of Limulus Amebocyte Lysate LAL test.

- Responsibility/accountability:

Microbiologist

4.0 Abbreviations and Definitions

- LAL ---- Limulus Amebocyte Lysate

- LRW --- LAL reagent water

- CSE --- Control Sample Endotoxin

- PPE --- Positive product control

- Procedure

- Reconstitution of lysate and CSE according to manufacturer’s recommendations

- Label five 12× 75mm dilution tubes A-E and LRW in the following proportions

- To tube A, add 1750µl of LRW

- To tube B, add 1800µl of LRW

- To tube C, add 1000µl of LRW

- To tube D, add 1000µl of LRW

- To tube E, add 1000µl of LRW

- Complete the dilution for each tube by adding the following

- To tube A, add 250µl of 25EU/ml CSE

- To tube B, add 200µl of solution in tube A

- To tube C, add 100µl of solution in tube B

- To tube D, add 100µl of solution in tube C

- To tube E, add 100µl of solution in tube D

NOTE:  vortex each tubes for at least 30secs.

- Label 4 tubes as sample1, sample 2, PPC 1, PPC2 and add 200µl of sample to each

NOTE: cover the reaction tubes with pyrogen free aluminum foil

- To each LRW tube, add 200µl of LRW

- To B1 & B2, add 100µl of solution in tube B

- To C1 & C2, add 100µl of solution in tube C

- To D1 & D2, add 100µl of solution in tube D

- To E1 & E2, add 100µl of solution in tube E

- Spike PPC labeled tubes with 10µl of tube A

- Add 100µl of reconstituted lysate to each of the reaction tubes

- Tap tubes gently to mix and incubate at 37˚c for exactly 1 hour

- After I hour, remove the tube from the heating block and slowly invert through 180˚c. A positive result indicated by the clot remaining in the bottom of tube through the invertion

NOTE: tube B (0.25EU/ml) always clot and tube E (0.03EU/ml) does not clot

- All the PPC samples must clot while the samples must not.                                                                Record the activity/values of the LAL results in the various products forms.

| S/no | Product | LAL Result Form No. |
| --- | --- | --- |
| 1 | Darrows full solution | BML-QA-STP-FPF-008-F06 |
| 2 | Isoplasma 4% solution | BML-QA-STP-FPF-014-F06 |
| 3 | Normal Saline (500ml) | BML-QA-STP-FPF-001-F06 |
| 4 | Dextrose 5% solution (500ml) | BML-QA-STP-FPF-003-F06 |
| 5 | Dextrose 5% Saline solution (500ml) | BML-QA-STP-FPF-002-F06 |
| 6 | Dextrose 4.3% saline | BML-QA-STP-FPF-012-F06 |
| 7 | Dextrose 10% solution | BML-QA-STP-FPF-013-F06 |
| 8 | Darrows half solution | BML-QA-STP-FPF-007-F06 |
| 9 | Mannitol 10% | BML-QA-STP-FPF-010-F06 |
| 10 | Mannitol 20% | BML-QA-STP-FPF-011-F06 |
| 11 | Ringers solution | BML-QA-STP-FPF-009-F06 |
| 12 | Biogyl solution | BML-QA-STP-FPF-015-F06 |
| 13 | Dextrose 50% solution | BML-QA-STP-FPF-018-F06 |
| 14 | Ciprofloxacin solution | BML-QA-STP-FPF-016-F06 |
| 15 | CAPD solution (1.5%) | BML-QA-STP-FPF-019-F06 |
| 16 | Fluconazole solution | BML-QA-STP-FPF-017-F06 |
| 17 | Normal Saline Solution (1000ml) | BML-QA-STP-FPF-004-F06 |
| 18 | Dextrose 5% solution (1000ml) | BML-QA-STP-FPF-006-F06 |
| 19 | Dextrose 5% Saline solution (1000ml) | BML-QA-STP-FPF-005-F06 |
| 20 | CAPD solution (1.5%) | BML-QA-STP-FPF-020-F06 |

- Materials and Equipment:

- Heating block

- 12 × 75mm dilution tubes

- 10 × 75mm dilution tubes

- Aluminum foil

- Micro pipette

6.0 REFERENCE/ENCLOSURE

LAL TEST RESULT FORMS- BML-QA-STP-FPF-(001…..020)-F06

- TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality control head |
| --- | --- |
| Quality Microbiology staff | Laboratory assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | MICROBIOLOGY LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |