- OBJECTIVE:

To provide written guidelines for the use of melting point apparatus

- SCOPE:

This SOP covers the written procedure to follow in the use of melting point apparatus.

- RESPONSIBILITY/ACCOUNTABILITY:

- The Chemical analyst shall carry out equipment calibration as at when due.

- The user reports any deviation or equipment malfunction.

- The equipment user must ensure that the equipment is always kept clean, especially the electrodes

- The Quality control Manager ensures external calibration is carried out as scheduled

4.0 ABBREVIATIONS AND DEFINITIONS

Melting Point: Temperature at which powder becomes liquid.

- PROCEDURE

- Clean the equipment always.

- Keep the apparatus on a table near electrical mains supply.

- Put some silicon oil in the beaker; fill it up to about half of its capacity.

- The silicon oil in the beaker should be heated with the aid of immersion heater fitted to the machine.

- Put some powder to be tested in the capillary and lightly tap it so that the powder is gathered at the bottom.

- Insert the capillary containing the powder through one of the small holes in the Teflon cover of the beaker.

- Keep the heater-controlling knob of the apparatus to its minimum; connect the cable to the mains 230volts 50Hz. AC supply. Turn the knob clockwise to put the mains and stirrer on.

- Insert the thermometer into the larger hole of the cover up to the bottom support, so that its bulb is well immersed in the oil.

- Watch through the magnifying lens fitted to the apparatus as the temperature rises.

- Watch and monitor the powder inside the capillary melting; notice the reading on the thermometer.

- The reading on the thermometer at the point of melting is the melting point of the powder.

- Upon completion, fill the melting point apparatus user logbook provided.

Melting point apparatus User Logbook LGB/QCS/CL/023/01

| Date | Used for | Start time | End time | Total time use | Status | Operator sign | Checked by | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Switch off the machine after use and clean up the equipment.

- REFERENCE/ENCLOSURE

- Beamex Melting point machine instruction manual.

- TRAINING INVOLVEMENT

| Quality Control Head | Chemical Analyst |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |