- OBJECTIVE

The objective of this SOP is to describe the Operation of Automatic Rotary Piston Filling/Capping Machine

2.0	SCOPE

This SOP provides written guidelines to follow the Operation of Automatic Rotary Piston Filling/Capping Machine

3.0	RESPONSIBILITY/ACCOUNTABILITY

- IPQC personnel are responsible for ensuring that this SOP is followed for the operation.

- The operator shall be responsible for the clear usage of the machine.

- The production supervisor should monitor them and give the necessary directives for effective operation of the machine.

- Production pharmacists shall ensure this procedure is followed effectively.

4.0 	DEFINITIONS AND ABBREVIATIONS

SOP - Standard Operating Procedure

QC/QA - Quality Control/Quality Assurance

ROPP - Rolls on Pilfer Proof

BMR - Batch Manufacturing Record

PPE - Personal Protective Equipment

LLC - Liquid Levels Control

5.0         PROCEDURE

Assembling & Set-Up

- Ensure that the Automatic Rotary Piston Filling/Capping Machine is cleaned as per SOP BML-PDS-PK-009

- Check for line clearance approval by QC/QA.

- Carry out all necessary settings like fixing of the filling nozzles, height and width of bottle guide plates on the conveyor with respect to bottle size, height of ROPP cap sealing head, height of main cap feeder pillar, machine speed, and speed of oriental bowl

- Adjust the bottle guide track so that the neck of the bottle moves below the nozzle freely.

- Ensure the position of the sensor is such that the light from the sensor falls on the neck of the bottles to fill.

- Adjust the volume of the liquid to be filled by raising or lowering the height of the syringes and/or the volume adjustment knob on the cam.

- Run the machine manually and confirm the fill volume of the liquid is in accordance with the specification

- Allow caps to reach the cap feeder through the cap elevator and continue running the machine manually to determine the capping integrity.

- Ensuring that all settings are OK, load the bottles on the turn table and run the machine.

Operation

- Ensure that the Bulk liquid is released by QC for filling, and the entire line is set and ready for operation.

- Fill the float tank automatically with bulk liquid by means of LLC valve button on the machine.

- Always run the machine with cleaned bottles.

- Ensure that the fill volume is checked regularly at the intervals specified in the BMR as in-process check and recorded.

- After completion of the batch / product, clean the machine.

Shutdown

- After completion of sealing operation switch “OFF” the machine.

- Ensure that no suspension product is left in the float tank except there is a breakdown.

- Put a proper Status Label

SAFETY, HEALTH AND ENVIRONMENT PRECAUTIONS

- Check for proper electrical connection.

- Before starting the operation, check for the correct direction of rotation by inch push button.

- Do not adjust the switches frequently.

- Always use hand gloves and nose masks during operation

- Ensure that the Bulk liquid is approved by QC for filling.

- REFERENCE/ENCLOSURE

- Nil

7.0 TRAINING INVOLVEMENT

| Production Manager | IPQC |
| --- | --- |
| Production Pharmacist | Operators |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | FILLING ROOM |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 01 | ### 00 | To Update |
| ### 02 | ### 01 | - Inclusion of enclosures format - Change in sop format generally |