- OBJECTIVE

The objective of this SOP is to describe the procedure to follow in cleaning and disinfecting the filling nozzles

2.0	SCOPE

This SOP applies to all filling nozzles in the Syrup manufacturing site of Biomedical Ltd.

3.0	RESPONSIBILITY/ACCOUNTABILITY

- IPQC personnel are responsible for ensuring that this SOP is followed for the cleaning before usage.

- The operator shall be responsible for the routine and in-depth cleaning of the filling nozzles.

- The production supervisor shall monitor the and give necessary directives for effective cleaning to be achieved.

- Production pharmacists shall ensure the cleaning is carried out and it’s effective

4.0 	DEFINITIONS AND ABBREVIATIONS

QA - Quality Assurance

DM - Demineralized

5.0	PROCEDURE

Precautions

- Be careful with machine parts when loosening and fixing

- Always use hand gloves and nose masks during operation

Cleaning

- Loose and remove the nozzles from the machine appropriately

- Manually clean the inside of the nozzles with soft brush and soap (12.5% Sodium laureth solution)

- Rinse the nozzles with clean DM water two times

Disinfection

Method 1

  - To 13L of DM water in a bucket, add about 50mls of 15% Chloral solution (hypochlorite solution)

  - Soak the already washed and rinsed nozzles in the water-chloral solution mixture in step 1 above for 5-10mins and remove the nozzles.

  - Keep the nozzles in hot DM water and leave for 3 minutes

  - Repeat the action in step 3 above one more time before finally rinsing in cold water.

  - Fix the nozzles on the machine and continue the machine cleaning operation.

Method 2

- Fill a 3L container with hot DM water (70-80oC)

- Deliver the already cleaned nozzles according to procedure described above into the hot DM water

- Cover the container tightly and allow the nozzles to sit in the hot DM water for 25-30minutes

- Remove the nozzles into another container filled with sizable quantity of IPA, allowing the IPA to fill each nozzle.

  - Fix the nozzles on the machine and continue the machine cleaning operation.

Note: When nozzle disinfection is required, Method 2 of disinfection needs no QC assessment of the nozzles after disinfection, since the disinfectant is water and leaves no residue on the nozzles.

SAFETY, HEALTH AND ENVIRONMENT PRECAUTIONS

- Always use appropriate PPE

- REFERENCE/ENCLOSURE

- Nil

7.0 TRAINING INVOLVEMENT

| Production Manager | IPQC |
| --- | --- |
| Production Pharmacist | Operators |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | FILLING ROOM |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 01 | ### 00 | To Update |
| ### 02 | ### 01 | - Inclusion of enclosures format - Change in sop format generally |