- OBJECTIVE

The objective of this SOP is to describe the Operation of an Automatic bottle labeling machine

- SCOPE

This SOP applies to the Automatic labelling machine on the NPM Automatic rotary filling/capping machine

- RESPONSIBILITY/ACCOUNTABILITY

- IPQC personnel are responsible for ensuring that this SOP is followed for the operation and cleaning before usage.

- The operator shall be responsible for cleaning and operation of the machine.

- The production supervisor shall monitor and give necessary directives for effective adherence to this SOP.

- Production pharmacists should ensure the cleaning and operations is carried out and it’s effective

- DEFINITIONS AND ABBREVIATIONS

SOP - Standard Operating Procedure

- PROCEDURE

Cleaning

- Clean the surfaces with soft cloth and isopropyl alcohol

- Clean the pressing belt with napkin

- Clean the coding point of the ink cartridge with lint-free cloth and isopropyl alcohol

- Clean the electrical part with dry napkin

- Remove any obvious stickers from all the surfaces

Settings

Setting of Conveyor guide plate for bottle size and height

- Set the conveyor guide plate for bottle size (horizontal setting) by loosening the knob

- Set the conveyor guide plate for bottle height (vertical setting) by loosening the Allen bolt

- Check that the bottles are in the center of conveyor slat chain

- Check the bottles easily slide between two guide plates.

- Check that all knobs are tightened properly

Setting of the pressing belt for Round Bottle

- Loosen the Allen bolt of the pressing belt

- Move the pressing belt to or fro according to the bottle size

- Retighten the Allen bolt after the setting is over

Setting of the bottle delay drive for round bottle

- Set the bottle delay drive and the guide for bottle size by loosening the Hex bolt that hold the ‘O’ Ring, Motor and the Gearbox. Retighten the hex bolt appropriately.

Setting the bottle delay drive for Flat bottle

- Set the two bottle delay drives for the bottle size by loosening the hex bolt holding the ‘O’ Ring, Motor and Gearbox. Hold the drives in place by tightening the hex bolt properly.

Setting of height of top pressing device unit for flat Bottle

- Turn the hand wheel in clockwise direction to move the flat device unit up. Turn the hand wheel in a counterclockwise direction to move the flat device unit down.

Fixing of Label

- Fix in the label/sticker roll and allow it to pass through the right channel from the Unwinder Assembly to the Sticker Roll Holding Assembly.

Setting of Dispenser height for change in sticker distance from the bottom of Bottle

- Turn the hand wheel in clockwise direction to move the flat device unit up

- Turn the hand wheel in counterclockwise direction to move the flat device unit down

Setting of Labelling character (Printer)

- Firstly, remove the flash drive from the printer and insert it into a computer which has the coding software (Design) installed in it.

- Save in the required codes needed for printing on the flash drive.

- Insert the cartridge appropriately into the cartridge port.

- Insert the flash drive into the printer, tap the printer power button to change the indicator from green to orange.

- Then, click on the numeric button on the printer to select the file number from the flash and then click on the power button again changing the indicator back to green, which implies the automatic saving of the file on the printer.

Operations

  - Having carried out all settings, load few filled and covered bottles on the conveyor and run at low speed to determine the correctness of the settings

  - Check the coding correctness, legibility and arrangement

  - Get the approval for the correct labelling from the Pharmacist and the QC/QA officer and affix the signed copy of the correct label on the BPR.

  - After due approval, continue running the labelling and constantly ensure correctness of the coding throughout the labelling activity.

Shutdown

- After completion of labeling operation switch “OFF” the machine.

- Ensure that no bottle is left in the labeling machine and no label is left in the label box.

- Remove the cartridge from the cartridge case and clean the print head with soft cloth. Keep the cartridge covered until next use.

SAFETY, HEALTH AND ENVIRONMENT PRECAUTIONS

* Check for proper electrical connection.

- REFERENCE/ENCLOSURE

- Nil

7.0 TRAINING INVOLVEMENT

| Production Manager | Production Operator |
| --- | --- |
| Production Pharmacist | Engineering maintenance officer |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | LABELING ROOM |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 01 | ### 00 | To Update |
| ### 02 | ### 01 | - Change in sop format generally |