- OBJECTIVE

The objective of this SOP is to describe the Cleaning of Automatic rotary bottle washing machine

2.0	SCOPE

This SOP provides written guidelines to follow the Cleaning of Automatic rotary bottle washing machine

3.0	RESPONSIBILITY/ACCOUNTABILITY

- The operator will carry out the cleaning according to this SOP.

- The production supervisor shall monitor and give necessary directives for effective cleaning of the machine

- Production pharmacists will ensure the operation is carried out and it’s effective

4.0        DEFINITIONS AND ABBREVIATIONS

SOP - Standard Operating Procedure

IPA - Isopropyl alcohol

DM - Demineralized

5.0	 PROCEDURE

Daily Cleaning

- Wash tank I and tank II with DM water and clean cloth daily and after every shift

- Wipe the inner surface of the tanks with isopropyl alcohol

- Rinse the tanks thoroughly with DM water to be used.

- Clean the conveyor and other parts of the machine with napkin moistened with water-IPA mixture

- Clean the electrical parts with dry napkin

Weekly Cleaning

- Wash tank I and tank II thoroughly with soap solution (12.5% sodium laureth sulphate)

- Rinse copiously with DM water

- Repeat all the activities for daily cleaning

Monthly Cleaning

- Repeat all the activities for weekly cleaning

- Change the polypropylene pleating filters.

SHUTDOWN

- After completion of washing operation switch “OFF” the machine & close all valves for Water line.

- Drain the water from the Tank I and II and ensure that no water is left inside the tank

SAFETY, HEALTH AND ENVIRONMENT PRECAUTIONS

- Check for proper electrical connection & water connections are in place & supplies are received.

- Watch for trip hazard when transferring washed bottles to the designated spot

- REFERENCE/ENCLOSURE

- Nil

7.0 TRAINING INVOLVEMENT

| Production Manager | IPQC |
| --- | --- |
| Production Pharmacist | Operators |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PROCESSING |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 01 | ### 00 | To Update |
| ### 02 | ### 01 | - Inclusion of enclosures format - Change in sop format generally |