- OBJECTIVE:

To provide written guidelines for cleaning and sanitization of clean area to maintain aseptic conditions and minimize contamination risks.

- SCOPE:

This SOP applies to Grade C areas in the sterile manufacturing department which includes cleanrooms, change rooms, lamina airflows (LAFs), pass boxes, equipment surfaces.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Housekeeping staff: execute cleaning as per schedule

- Production Operator: perform equipment and immediate area cleaning.

- QA : monitor and verify compliance

- Engineering: ensure HVAC system supports cleanliness.

- ABBREVIATION AND DEFINITIONS

- Sterile Area: controlled environment designed to prevent microbial contamination.

- Grade C Area: cleanroom classifications based on particle count and microbial limits.

- Disinfectant: a validated agent used to destroy microorganisms.

- Residue Removal: the process of removing disinfectant residues after application.

- MATERIALS REQUIRED

- Validated disinfectants ( e.g. 70%IPA,

- Water for injection/ Purified water.

- Sterile mops and Mop handles

- Non-shedding, lint -free wipes

- Fogger (if applicable)

- PPE, sterile gloves, goggles, face mask

- Cleaning and sanitization checklists

- FREQUENCY OF CLEANING

Grade C- Twice daily

Change rooms – once per shift

Equipment surfaces: before and after use

- PROCEDURES

PRECAUTIONS

- The alternative use of two or more disinfectant is recommended at regular interval to prevent proliferation of resistant strain of microorganisms.

- The alternative should be of different chemicals type and preferably possess different spectrum of anti-microbial activity.

- Disinfectants of different chemical types should not be mixed and disinfectants should not be mixed with cleaning agents. Mixing can result in drastic reduction in antimicrobial activity.

- Disinfectant and cleaning agents should be freshly prepared .Only water for injection should be used for dilution as per need.

- Aqueous dilutions should not be stored overnight as microorganisms can grow on storage

STEP BY STEP CLEANING

(A.)FLOOR CLEANING

- Mop with sterile water first , followed by disinfectant ( e.g 70% IPA)

- Change mop heads after each room or area

- Record in the floor cleaning log

(B) WALL AND CEILING CLEANING)

- Wipe using a lint –free cloth soaked in disinfectant

- Start from the top and move down in vertical strokes

- Clean corners, joints, and behind equipment regularly

(C) RABs, PASS BOXES

- Clean all internal surfaces with 70% IPA or designated disinfectant.

- Wipe in one direction, avoid circular movements.

- Clean before each batch and between shifts

(D) EQUIPMENT SURFACES

- Wipe with sterile 70% IPA before and after each batch.

- Ensure residue free drying.

(E) FOGGING ( IF APPLICABLE)

- Use approved fogging agent( e.g, hydrogen peroxide) weekly or as per SOP

- Record fogging in logbook with fogger ID, agent and concentration

SAFETY

- Always wear sterile PPE

- Never re-use mops or wipes across different grades

- Use only validated disinfectants

- Report any deviation or spillage to QA

- At no instance of time there should be water or product accumulation on the floor.

- At the end of the day, drain the excess solution of cleaning agents and disinfectant in the drain line wash and dry the empty container

- Mop the working area with 70% Isopropyl alcohol daily after completion of the work

- Fumigate the clean room areas, with Fogger machine on the week end after work is over.

- Record it in a cleaning record as per annexure.

SCHEDULE FOR DISINFECTANTS

| DAY | DISINFECTANT | CONCENTRATION |
| --- | --- | --- |
| MON | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| TUE | HYPO (Hypochlorite Solution) | 3.0% v/v |
| WED | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| THU | HYPO (Hypochlorite Solution) | 3.0% v/v |
| FRI | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| SAT | PURIT(Cetrimide Solution) | 2.0% v/v |

- Materials, Equipment, Apparatus and Reagents

- PPE

- Hand gloves

- Nose Mask

- REFERENCE AND ENCLOSURE

ANNEXURE1 – Daily cleaning Checklists

ANNEXURE 3: Fogging Record Template

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |

ANNEXURE 1

DAILY CLEANING WEEK OF _________________SIGNATURE________________DATE_________

| FILLING ROOM./BATCH CODING UNIT | M | T | W | TH | F | REMARKS |
| --- | --- | --- | --- | --- | --- | --- |
| Panels |  |  |  |  |  |  |
| # Floor |  |  |  |  |  |  |
| # Ceiling light covers |  |  |  |  |  |  |
| # Ceilings |  |  |  |  |  |  |
| # Dynamic pass box |  |  |  |  |  |  |
| # Air shower door |  |  |  |  |  |  |
| # Air shower door handle |  |  |  |  |  |  |
| # Stainless Piping |  |  |  |  |  |  |
| # Stainless stood |  |  |  |  |  |  |
| # Stainless bucket |  |  |  |  |  |  |
| # Stainless carts |  |  |  |  |  |  |
| # Light switches |  |  |  |  |  |  |
| # Exhaust lines |  |  |  |  |  |  |
| # Intercom |  |  |  |  |  |  |
| # Digital hygrometers |  |  |  |  |  |  |
| # Wall clocks |  |  |  |  |  |  |
| # CCTV camera |  |  |  |  |  |  |
| # Filling machines/nozzles |  |  |  |  |  |  |
| # Light sockets |  |  |  |  |  |  |
| # Stainless pallet |  |  |  |  |  |  |
| # Stainless buckets |  |  |  |  |  |  |
| # Plastic buckets |  |  |  |  |  |  |
| # Baskets |  |  |  |  |  |  |

Cleaning personnel:                                                                                     Signature                                        Date

Prod. Supervisor   :                                                                                      Signature                                         Date

QA Officer                                                                                                      Signature                                        Date