- OBJECTIVE:

To define the procedure for carrying out batch-to-batch changeover of the same product in the manufacturing area to ensure that the process area and equipment are properly cleaned, identified, and documented before starting the next batch, in compliance with Good Manufacturing Practices (GMP).

- SCOPE:

This SOP applies to all production areas involved in the manufacturing and packaging of pharmaceutical products within Biomedical Ltd specifically when changing over from one batch of a product to another batch of the same product and same strength.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Production supervisor: To perform batch-to-batch changeover as per this SOP and record all activities.

- In-Process Quality Assurance (IPQA): To verify and approve the line and area before the next batch starts.

- Engineering Officer: To ensure equipment is operational and perform maintenance if required.

- Cleaning Personnel: To carry out cleaning as per cleaning procedure.

- ABBREVIATION AND DEFINITIONS

DEFINITIONS

- Changeover: The process of preparing and clearing the production area and equipment between two batches.

- Batch-to-Batch Changeover: Changeover between two consecutive batches of the same product, same strength, and same dosage form.

- PROCEDURES

BATCH TO BATCH CHANGEOVERS

Pre-Cleaning Activities

- Ensure that the previous batch production is completed.

- Remove all balance materials, waste, and labels related to the previous batch

- Return all remaining materials to the dispensing area or warehouse as per SOP.

- Record the end of batch in the equipment usage logbook.

Cleaning of Area and Equipment

- Perform cleaning as per respective Equipment Cleaning SOP.

- Use approved cleaning agents and purified water for wiping all contact parts.

- Clean floors, walls, and surfaces of any material residues.

- Inspect equipment visually to ensure cleanliness.

- Attach “Cleaned” label with date, time, and signature after cleaning.

Line Clearance and Verification

- The production personnel shall inform QA that cleaning is completed and request for line clearance.

- QA shall inspect, the equipment cleanliness, removal of old labels, materials, and documents, correct status labels affixed, proper documentation in logbooks

- QA signs off the Batch-to-Batch Changeover Checklist after verification.

Documentation

- Record all cleaning and inspection details in:

- Equipment Logbook

- Area Logbook

- Changeover Checklist

- Cross-check and attach with the batch manufacturing record (BMR).

Start of Next Batch

- Only after QA approval and documentation, start the next batch.

- Record the time and date of changeover completion in the BMR.

Precautions

- Do not start next batch production without QA line clearance.

- Ensure all rejected materials and previous batch residues are removed.

- Use dedicated cleaning tools and avoids cross-contamination.

Materials, Equipment, Apparatus and Reagents

- Cleaning tools and materials (lint-free cloths, approved detergents, purified water, etc.)

- Area and equipment logbooks

- Changeover checklist (BML-QDG-QA-027-F01)

- Labels: “Cleaned”, “Under Cleaning”, “To Be Cleaned”, etc.

- REFERENCE AND ENCLOSURE

WHO TRS 986 Annex 2 – Good Manufacturing Practices for Pharmaceutical Products

EU GMP Guidelines, Part I, Chapter 5 – Production

Internal SOPs on Equipment and Area Cleaning

Batch-to-Batch Changeover Checklist- BML-QDG-QA-027-F01

- TRAINING INVOLVEMENT

| Quality and Production Managers | IPQA heads |
| --- | --- |
| QA staff | Production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |