- OBJECTIVE:

This SOP is prepared to have a laid down standard procedure for operation, calibration and cleaning of Flame Photometer.

- SCOPE:

This SOP is applicable for Cleaning and calibration of Flame Photometer used for Measuring of various samples in Production Department.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Quality assurance Officer, Quality Control Analyst, Quality Assurance Manager and Superintendent Pharmacist.

- ABBREVIATION AND DEFINITIONS

Nil

- PROCEDURES

- Switch on the instrument from the mains.

- Turn on the clean, pulse free air at a pressure of between 15 – 20 Psi

- Switch the instrument on by depressing the “POWER” switch.

- Turn on the fuel-knob to release the fuel.

- Depress ignition switch and hold it until the FLM is illuminated.

- the filter select control to the desired position

- Aspirate distilled water and set the reading to zero by adjusting the “blank” control.

- Allow the instrument to warm up for 15 – 30 minutes

- Aspirate the standard solution that has a slightly higher concentration than that expected sample-awaiting analysis.

- Aspirate the standard solution and take their readings.

- Aspirate the sample to be analyzed and take the reading.

- REFERENCE AND ENCLOSURE

In-House Method

7.0 TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality Control Head |
| --- | --- |
| QA Doc. Unit staff | All QC staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |